# 10	Physical uplink control channel procedures

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms ‘secondary cell’, ‘secondary cells’ , ‘serving cell’, ‘serving cells’ in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms ‘secondary cell’, ‘secondary cells’, ‘serving cell’, ‘serving cells’ in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term ‘primary cell’ in this clause refers to the PSCell of the SCG.

## 10.1	UE procedure for determining physical uplink control channel assignment

If the UE is configured for a single serving cell and is not configured for simultaneous PUSCH and PUCCH transmissions, then in subframe  uplink control information (UCI) shall be transmitted

- on PUCCH using format 1/1a/1b/3 or 2/2a/2b if the UE is not transmitting PUSCH

- on PUSCH if the UE is transmitting PUSCH in subframe  unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, in which case UCI is not transmitted

If the UE is configured for a single serving cell and simultaneous PUSCH and PUCCH transmission, then in subframe UCI shall be transmitted

- on PUCCH using format 1/1a/1b/3 if the UCI consists only of HARQ-ACK and/or SR

- on PUCCH using format 2 if the UCI consists only of periodic CSI

- on PUCCH using format 2/2a/2b/3 if the UCI consists of periodic CSI and HARQ-ACK and if the UE is not transmitting PUSCH

- on PUCCH and PUSCH if the UCI consists of HARQ-ACK/HARQ-ACK+SR/positive SR and periodic/aperiodic CSI and if the UE is transmitting PUSCH in subframe n, in which case the HARQ-ACK/HARQ-ACK+SR/positive SR is transmitted on PUCCH using format 1/1a/1b/3 and the periodic/aperiodic CSI transmitted on PUSCH unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, in which case periodic/aperiodic CSI is not transmitted

If the UE is configured with more than one serving cell and is not configured for simultaneous PUSCH and PUCCH transmission, then in subframe  UCI shall be transmitted

- on PUCCH using format 1/1a/1b/3 or 2/2a/2b if the UE is not transmitting PUSCH

- on PUSCH of the serving cell given in subclause 7.2.1 if the UCI consists of aperiodic CSI or aperiodic CSI and HARQ-ACK

- on primary cell PUSCH if the UCI consists of periodic CSI and/or HARQ-ACK and if the UE is transmitting on the primary cell PUSCH in subframe  unless the primary cell PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, in which case UCI is not transmitted

- on PUSCH of the secondary cell with smallest SCellIndex if the UCI consists of periodic CSI and/or HARQ-ACK and if the UE is not transmitting PUSCH on primary cell but is transmitting PUSCH on at least one secondary cell

If the UE is configured with more than one serving cell and simultaneous PUSCH and PUCCH transmission, then in subframe  UCI shall be transmitted

- on PUCCH using format 1/1a/1b/3 if the UCI consists only of HARQ-ACK and/or SR

- on PUCCH using format 2 if the UCI consists only of periodic CSI

- as described in subclause 10.1.1, if the UCI consists of periodic CSI and HARQ-ACK and if the UE is not transmitting on PUSCH

- on PUCCH and primary cell PUSCH if the UCI consists of HARQ-ACK and periodic CSI and the UE is transmitting PUSCH on the primary cell, in which case the HARQ-ACK is transmitted on PUCCH using format 1a/1b/3 and the periodic CSI is transmitted on PUSCH unless the primary cell PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, in which case periodic CSI is not transmitted

- on PUCCH and PUSCH of the secondary cell with the smallest SCellIndex if the UCI consists of HARQ-ACK and periodic CSI and if the UE is not transmitting PUSCH on primary cell but is transmitting PUSCH on at least one secondary cell, in which case, the HARQ-ACK is transmitted on PUCCH using format 1a/1b/3 and the periodic CSI is transmitted on PUSCH

- on PUCCH and PUSCH if the UCI consists of HARQ-ACK/HARQ-ACK+SR/positive SR and aperiodic CSI in which case the HARQ-ACK/HARQ-ACK+SR/positive SR is transmitted on PUCCH using format 1/1a/1b/3 and the aperiodic CSI is transmitted on PUSCH of the serving cell given in subclause 7.2.1

If the UE is configured with more than one serving cell, then reporting prioritization and collision handling of periodic CSI reports of a certain PUCCH reporting type is given in subclause 7.2.2.

A UE transmits PUCCH only on the primary cell.

A UE is configured by higher layers to transmit PUCCH on one antenna port  or two antenna ports .

For FDD or FDD-TDD and primary cell frame structure 1, with two configured serving cells and PUCCH format 1b with channel selection or for FDD with two or more configured serving cells and PUCCH format 3,  where  is the number of configured cells and  is the number of transport blocks or the SPS release PDCCH/EPDCCH, if any, received in subframe  in serving cell .

For TDD and a UE not configured with the parameter EIMTA-MainConfigServCell-r12 for any serving cell, if a UE is configured with one serving cell, or the UE is configured with more than one serving cell and the UL/DL configurations of all serving cells are the same, then

-	For TDD with two configured serving cells and PUCCH format 1b with channel selection and a subframe n with M = 1, or for TDD UL/DL configuration 0 and PUCCH format 3, , where is the number of transport blocks or the SPS release PDCCH/EPDCCH, if any, received in subframe  in serving cell , where , and M is the number of elements in K.

-	For TDD UL/DL configurations 1-6 and PUCCH format 3, or for TDD with two configured serving cells and PUCCH format 1b with channel selection and M = 2,  where  is the  in serving cell ,  is the  in serving cell , and  is the number of HARQ-ACK bits corresponding to the configured DL transmission mode on serving cell . In case spatial HARQ-ACK bundling is applied,  and  is the number of PDCCH/EPDCCH or PDSCH without a corresponding PDCCH/EPDCCH received in subframe  and serving cell, where  and M is the number of elements in K. In case spatial HARQ-ACK bundling is not applied, is the number of transport blocks received or the SPS release PDCCH/EPDCCH received in subframe  in serving cell , where  and M is the number of elements in K. =0 if no transport block or SPS release PDCCH/EPDCCH is detected in subframe(s)  in serving cell , where .

-	For TDD with two configured serving cells and PUCCH format 1b with channel selection and M = 3 or 4,  if UE receives PDSCH or PDCCH/EPDCCH indicating downlink SPS release only on one serving cell within subframes ,where ;otherwise .

For TDD if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations, or if the UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, or for FDD-TDD and primary cell frame structure 2, then

- For PUCCH format 3, or for two configured serving cells and PUCCH format 1b with channel selection and  (defined in subclause 10.1.3.2.1 for TDD and subclause 10.1.3A for FDD-TDD),  where  is the  in serving cell ,  is the  in serving cell , and  is the number of HARQ-ACK bits corresponding to the configured DL transmission mode on serving cell . In case spatial HARQ-ACK bundling is applied,  and  is the number of PDCCH/EPDCCH or PDSCH without a corresponding PDCCH/EPDCCH received in subframe  and serving cell, where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD). In case spatial HARQ-ACK bundling is not applied, is the number of transport blocks received or the SPS release PDCCH/EPDCCH received in subframe  in serving cell , where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD). =0 if no transport block or SPS release PDCCH/EPDCCH is detected in subframe(s)  in serving cell , where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD). For a serving cell , set if the DL-reference UL/DL configuration (defined in subclause 10.2) for serving cell   is TDD UL/DL configuration 0,

- For two configured serving cells and PUCCH format 1b with channel selection and M = 3 or 4 (defined in subclause 10.1.3.2.1 for TDD and subclause 10.1.3A for FDD-TDD),  if UE receives PDSCH or PDCCH/EPDCCH indicating downlink SPS release only on one serving cell within subframes ,where  and (defined in subclause 7.3.2.2 for TDD and subclause 7.3.4 for FDD-TDD); otherwise .

Throughout the following subclauses, subframes are numbered in monotonically increasing order; if the last subframe of a radio frame is denoted as , the first subframe of the next radio frame is denoted as .

Throughout the following subclauses, if the UE is configured with higher layer parameter n1PUCCH-AN-r11 then  is given by n1PUCCH-AN-r11, else  is given by higher layer parameter n1PUCCH-AN .

### 10.1.1	PUCCH format information

Using the PUCCH formats defined in subclause 5.4.1 and 5.4.2 in [3], the following combinations of UCI on PUCCH are supported:

- Format 1a for 1-bit HARQ-ACK or in case of FDD or FDD-TDD primary cell frame structure type 1 for 1-bit HARQ-ACK with positive SR.

- Format 1b for 2-bit HARQ-ACK or for 2-bit HARQ-ACK with positive SR.

- Format 1b for up to 4-bit HARQ-ACK with channel selection when the UE is configured with more than one serving cell or, in the case of TDD, when the UE is configured with a single serving cell.

- Format 1 for positive SR.

- Format 2 for a CSI report when not multiplexed with HARQ-ACK.

- Format 2a for a CSI report multiplexed with 1-bit HARQ-ACK for normal cyclic prefix.

- Format 2b for a CSI report multiplexed with 2-bit HARQ-ACK for normal cyclic prefix.

- Format 2 for a CSI report multiplexed with HARQ-ACK for extended cyclic prefix.

- Format 3 for up to 10-bit HARQ-ACK for FDD or FDD-TDD primary cell frame structure type 1 and for up to 20-bit HARQ-ACK for TDD and for up to 21 bit HARQ-ACK for FDD-TDD primary cell frame structure type 2.

- Format 3 for up to 11-bit corresponding to 10-bit HARQ-ACK and 1-bit positive/negative SR for FDD or FDD-TDD and for up to 21-bit corresponding to 20-bit HARQ-ACK and 1-bit positive/negative SR for TDD and for up to 22-bit corresponding to 21-bit HARQ-ACK and 1-bit positive/negative SR for FDD-TDD primary cell frame structure type 2.

- Format 3 for HARQ-ACK, 1-bit positive/negative SR (if any) and a CSI report for one serving cell.

For a UE configured with PUCCH format 3 and HARQ-ACK transmission on PUSCH or using PUCCH format 3, or for a UE configured with two serving cells and PUCCH format 1b with channel selection and HARQ-ACK transmission on PUSCH, or for UE configured with one serving cell and PUCCH format 1b with channel selection according to Tables 10.1.3-5, 10.1.3-6, 10.1.3-7 and HARQ-ACK transmission on PUSCH:

-	if the configured downlink transmission mode for a serving cell supports up to 2 transport blocks and only one transport block is received in a subframe, the UE shall generate a NACK for the other transport block if spatial HARQ-ACK bundling is not applied.

-	if neither PDSCH nor PDCCH/EPDCCH indicating downlink SPS release is detected in a subframe for a serving cell, the UE shall generate two NACKs when the configured downlink transmission mode supports up to 2 transport blocks and the UE shall generate a single NACK when the configured downlink transmission mode supports a single transport block.

The scrambling initialization of PUCCH format 2, 2a, 2b and 3 is by C-RNTI.

For a UE that is configured with a single serving cell and is not configured with PUCCH format 3, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH, the periodic CSI report is multiplexed with HARQ-ACK on PUCCH if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE, otherwise the CSI is dropped.

For TDD and for a UE that is configured with a single serving cell and with PUCCH format 3, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH, if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE or if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE, the periodic CSI report is multiplexed with HARQ-ACK or dropped as described in subclause 7.3, otherwise the CSI is dropped.

For FDD or for FDD-TDD and primary cell frame structure type 1 and for a UE that is configured with more than one serving cell, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH,

-	if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE and if the HARQ-ACK corresponds to a PDSCH transmission or PDCCH/EPDCCH indicating downlink SPS release only on the primary cell,

then the periodic CSI report is multiplexed with HARQ-ACK on PUCCH using PUCCH format 2/2a/2b

-	else if the UE is configured with PUCCH format 3 and if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE, and if PUCCH resource is determined according to subclause 10.1.2.2.2, and

-	if the total number of bits in the subframe corresponding to HARQ-ACKs, SR (if any), and the CSI is not larger than 22 or

-	if the total number of bits in the subframe corresponding to spatially bundled HARQ-ACKs, SR (if any), and the CSI is not larger than 22

then the periodic CSI report is multiplexed with HARQ-ACK on PUCCH using the determined PUCCH format 3 resource according to [4]

-	otherwise,

CSI is dropped.

For TDD or for FDD-TDD and primary cell frame structure type 2 and for a UE that is configured with more than one serving cell, in case of collision between a periodic CSI report and an HARQ-ACK in a same subframe without PUSCH, if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE or if the parameter simultaneousAckNackAndCQI-Format3-r11 provided by higher layers is set TRUE, the periodic CSI report is multiplexed with HARQ-ACK or dropped as described in subclause 7.3, otherwise the CSI is dropped.

In case of collision between a periodic CSI report and an HARQ-ACK in a same subframe with PUSCH, the periodic CSI is multiplexed with the HARQ-ACK in the PUSCH transmission in that subframe if the UE is not configured by higher layers for simultaneous PUCCH and PUSCH transmissions. Otherwise, if the UE is configured by higher layers for simultaneous PUCCH and PUSCH transmissions, the HARQ-ACK is transmitted in the PUCCH and the periodic CSI is transmitted in the PUSCH.

If each of the serving cells configured for the UE has frame structure type 1, UE procedures for HARQ-ACK feedback are given in subclause 10.1.2.

If each of the serving cells configured for the UE has frame structure type 2, UE procedures for HARQ-ACK feedback are given in subclause 10.1.3.

If the UE is configured for more than one serving cell, and if the frame structure type of any two configured serving cells is different, and if the primary cell is frame structure type 1, UE procedure for HARQ-ACK feedback is given in subclause 10.1.2A.

If the UE is configured for more than one serving cell, and if the frame structure type of any two configured serving cells is different, and if the primary cell is frame structure type 2, UE procedure for HARQ-ACK feedback is given in subclause 10.1.3A.

### 10.1.2	FDD HARQ-ACK feedback procedures

For FDD and for a UE transmitting HARQ-ACK using PUCCH format 1b with channel selection or PUCCH format 3, the UE shall determine the number of HARQ-ACK bits, ,  based on the number of configured serving cells and the downlink transmission modes configured for each serving cell. The UE shall use two HARQ-ACK bits for a serving cell configured with a downlink transmission mode that support up to two transport blocks; and one HARQ-ACK bit otherwise.

A UE that supports aggregating at most 2 serving cells with frame structure type 1 shall use PUCCH format 1b with channel selection for transmission of HARQ-ACK when configured with more than one serving cell with frame structure type 1.

A UE that supports aggregating more than 2 serving cells with frame structure type 1 is configured by higher layers to use either PUCCH format 1b with channel selection or PUCCH format 3 for transmission of HARQ-ACK when configured with more than one serving cell with frame structure type 1.

The FDD HARQ-ACK feedback procedure for one configured serving cell is given in subclause 10.1.2.1 and procedures for more than one configured serving cell are given in subclause 10.1.2.2.

#### 10.1.2.1	FDD HARQ-ACK procedure for one configured serving cell

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1a/1b.

For FDD and one configured serving cell, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subframe  for  mapped to antenna port p for PUCCH format 1a/1b [3], where

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH in subframe  , or for a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe  , the UE shall use  for antenna port , where  is the number of the first CCE (i.e. lowest CCE index used to construct the PDCCH) used for transmission of the corresponding DCI assignment and  is configured by higher layers. For two antenna port transmission the PUCCH resource for antenna port  is given by .

- for a PDSCH transmission on the primary cell where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2. For a UE configured for two antenna port transmission, a PUCCH resource value in Table 9.2-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

- for a PDSCH transmission indicated by the detection of a corresponding EPDCCH in subframe  , or for an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe  , the UE shall use

  - if EPDCCH-PRB-set is configured for distributed transmission

  - if EPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set is given in subclause 6.8A.1 in [3], is determined from the antenna port used for localized EPDCCH transmission which is described in subclause 6.8A.5 in [3].  For two antenna port transmission the PUCCH resource for antenna port  is given by

  - if EPDCCH-PRB-set is configured for distributed transmission

  - if EPDCCH-PRB-set is configured for localized transmission

Table 10.1.2.1-1: Mapping of ACK/NACK Resource offset Field 
in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D to values

| ACK/NACK Resource offset field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D |  |
| --- | --- |
| 0 | 0 |
| 1 | -1 |
| 2 | -2 |
| 3 | 2 |

#### 10.1.2.2	FDD HARQ-ACK procedures for more than one configured serving cell

The FDD HARQ-ACK feedback procedures for more than one configured serving cell are either based on a PUCCH format 1b with channel selection HARQ-ACK procedure as described in subclause 10.1.2.2.1 or a PUCCH format 3 HARQ-ACK procedure as described in subclause 10.1.2.2.2.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 3.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1b with channel selection and FDD with two configured serving cells.

##### 10.1.2.2.1	PUCCH format 1b with channel selection HARQ-ACK procedure

For two configured serving cells and PUCCH format 1b with channel selection, the UE shall transmit  on PUCCH resource  for mapped to antenna port p using PUCCH format 1b where

-	 for antenna port  where  is selected from  PUCCH resources, where  and, according to Table 10.1.2.2.1-3, Table 10.1.2.2.1-4, Table 10.1.2.2.1-5 in subframe .  HARQ-ACK(j) denotes the ACK/NACK/DTX response for a transport block or SPS release PDCCH/EPDCCH associated with serving cell , where the transport block and serving cell for HARQ-ACK(j) and  PUCCH resources are given by Table 10.1.2.2.1-1.

-	 for antenna port, where  is selected from  PUCCH resources, configured by higher layers where  and, according to Table 10.1.2.2.1-3, Table 10.1.2.2.1-4, Table 10.1.2.2.1-5 by replacing with and replacing with  in subframe , when the UE is configured with two antenna port transmission for PUCCH format 1b with channel selection.

A UE configured with a transmission mode that supports up to two transport blocks on serving cell, , shall use the same HARQ-ACK response for both the transport blocks in response to a PDSCH transmission with a single transport block or a PDCCH/EPDCCH indicating downlink SPS release associated with the serving cell .

Table 10.1.2.2.1-1: Mapping of Transport Block and Serving Cell to HARQ-ACK(j)
 for PUCCH format 1b HARQ-ACK channel selection

| A | HARQ-ACK(j) |  |  |  |
| --- | --- | --- | --- | --- |
|  | HARQ-ACK(0) | HARQ-ACK(1) | HARQ-ACK(2) | HARQ-ACK(3) |
| 2 | TB1 Primary cell | TB1 Secondary cell | NA | NA |
| 3 | TB1 Serving cell1 | TB2 Serving cell1 | TB1 Serving cell2 | NA |
| 4 | TB1 Primary cell | TB2 Primary cell | TB1 Secondary cell | TB2 Secondary cell |

The UE shall determine the  PUCCH resources,  associated with HARQ-ACK(j) where  in Table 10.1.2.2.1-1, according to

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH in subframe  on the primary cell, or for a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe  on the primary cell, the PUCCH resource is , and for transmission mode that supports up to two transport blocks, the PUCCH resource  is given by  where  is the number of the first CCE used for transmission of the corresponding PDCCH and  is configured by higher layers.

- for a PDSCH transmission on the primary cell where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2. For transmission mode that supports up to two transport blocks, the PUCCH resource  is given by

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH/EPDCCH in subframe  on the secondary cell, the value of , and the value of  for the transmission mode that supports up to two transport blocks  is determined according to higher layer configuration and Table 10.1.2.2.1-2. The TPC field in the DCI format of the corresponding PDCCH/EPDCCH shall be used to determine the PUCCH resource values from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.1-2. For a UE configured for a transmission mode that supports up to two transport blocks a PUCCH resource value in Table 10.1.2.2.1-2 maps to two PUCCH resources , otherwise, the PUCCH resource value maps to a single PUCCH resource .

- for a PDSCH transmission indicated by the detection of a corresponding EPDCCH in subframe  on the primary cell, or for an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe  on the primary cell, the PUCCH resource is given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set is given in subclause 6.8A.1 in [3], is determined from the antenna port used for localized EPDCCH transmission which is described in subclause 6.8A.5 in [3].

For transmission mode that supports up to two transport blocks, the PUCCH resource  is given by

-	 if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

Table 10.1.2.2.1-2: PUCCH Resource Value for HARQ-ACK Resource for PUCCH

| Value of 'TPC command for PUCCH' | or |
| --- | --- |
| '00' | The 1st PUCCH resource value configured by the higher layers |
| '01' | The 2nd PUCCH resource value configured by the higher layers |
| '10' | The 3rd PUCCH resource value configured by the higher layers |
| '11' | The 4th PUCCH resource value configured by the higher layers |
| NOTE: are determined from the first and second PUCCH resource lists configured by n1PUCCH-AN-CS-List-r10 in [11], respectively. |  |

Table 10.1.2.2.1-3: Transmission of Format 1b HARQ-ACK channel selection for

| HARQ-ACK(0) | HARQ-ACK(1) |  |  |
| --- | --- | --- | --- |
| ACK | ACK |  | 1,1 |
| ACK | NACK/DTX |  | 1,1 |
| NACK/DTX | ACK |  | 0,0 |
| NACK | NACK/DTX |  | 0,0 |
| DTX | NACK/DTX | No Transmission |  |

Table 10.1.2.2.1-4: Transmission of Format 1b HARQ-ACK channel selection for

| HARQ-ACK(0) | HARQ-ACK(1) | HARQ-ACK(2) |  |  |
| --- | --- | --- | --- | --- |
| ACK | ACK | ACK |  | 1,1 |
| ACK | NACK/DTX | ACK |  | 1,0 |
| NACK/DTX | ACK | ACK |  | 0,1 |
| NACK/DTX | NACK/DTX | ACK |  | 1,1 |
| ACK | ACK | NACK/DTX |  | 1,1 |
| ACK | NACK/DTX | NACK/DTX |  | 1,0 |
| NACK/DTX | ACK | NACK/DTX |  | 0,1 |
| NACK/DTX | NACK/DTX | NACK |  | 0,0 |
| NACK | NACK/DTX | DTX |  | 0,0 |
| NACK/DTX | NACK | DTX |  | 0,0 |
| DTX | DTX | DTX | No Transmission |  |

Table 10.1.2.2.1-5: Transmission of Format 1b HARQ-ACK channel selection for

| HARQ-ACK(0) | HARQ-ACK(1) | HARQ-ACK(2) | HARQ-ACK(3) |  |  |
| --- | --- | --- | --- | --- | --- |
| ACK | ACK | ACK | ACK |  | 1,1 |
| ACK | NACK/DTX | ACK | ACK |  | 0,1 |
| NACK/DTX | ACK | ACK | ACK |  | 0,1 |
| NACK/DTX | NACK/DTX | ACK | ACK |  | 1,1 |
| ACK | ACK | ACK | NACK/DTX |  | 1,0 |
| ACK | NACK/DTX | ACK | NACK/DTX |  | 0,0 |
| NACK/DTX | ACK | ACK | NACK/DTX |  | 0,0 |
| NACK/DTX | NACK/DTX | ACK | NACK/DTX |  | 1,0 |
| ACK | ACK | NACK/DTX | ACK |  | 1,1 |
| ACK | NACK/DTX | NACK/DTX | ACK |  | 1,0 |
| NACK/DTX | ACK | NACK/DTX | ACK |  | 0,1 |
| NACK/DTX | NACK/DTX | NACK/DTX | ACK |  | 0,0 |
| ACK | ACK | NACK/DTX | NACK/DTX |  | 1,1 |
| ACK | NACK/DTX | NACK/DTX | NACK/DTX |  | 1,0 |
| NACK/DTX | ACK | NACK/DTX | NACK/DTX |  | 0,1 |
| NACK/DTX | NACK | NACK/DTX | NACK/DTX |  | 0,0 |
| NACK | NACK/DTX | NACK/DTX | NACK/DTX |  | 0,0 |
| DTX | DTX | NACK/DTX | NACK/DTX | No Transmission |  |

##### 10.1.2.2.2	PUCCH format 3 HARQ-ACK procedure

For PUCCH format 3, the UE shall use PUCCH resource  or  for transmission of HARQ-ACK in subframe  for  mapped to antenna port p where

- for a PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH in subframe , or for a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe  on the primary cell,  the UE shall use PUCCH format 1a/1b and PUCCH resource  with  for antenna port , where  is the number of the first CCE (i.e. lowest CCE index used to construct the PDCCH) used for transmission of the corresponding PDCCH and  is configured by higher layers. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for antenna port  is given by .

- for a PDSCH transmission only on the primary cell where there is not a corresponding PDCCH/EPDCCH detected in subframe , the UE shall use PUCCH format 1a/1b and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 9.2-2. For a UE configured for two antenna port transmission for PUCCH format 1a/1b, a PUCCH resource value in Table 9.2-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

- for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding PDCCH/EPDCCH in subframe , the UE shall use PUCCH format 3 and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 10.1.2.2.2-1. 
The TPC field in the DCI format of the corresponding PDCCH/EPDCCH shall be used to determine the PUCCH resource values from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. For a UE configured for two antenna port transmission for PUCCH format 3, a PUCCH resource value in Table 10.1.2.2.2-1 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port . A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted in each DCI format of the corresponding secondary cell PDCCH/EPDCCH assignments in a given subframe.

- for a PDSCH transmission only on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , or for a EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe  on the primary cell, the UE shall use PUCCH format 1a/1b and PUCCH resource  given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set is given in subclause 6.8A.1 in [3], is determined from the antenna port used for localized EPDCCH transmission which is described in subclause 6.8A.5 in [3]. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for antenna port  is given by.

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

Table 10.1.2.2.2-1: PUCCH Resource Value for HARQ-ACK Resource for PUCCH

| Value of 'TPC command for PUCCH' or 'HARQ-ACK resource offset' |  |
| --- | --- |
| '00' | The 1st PUCCH resource value configured by the higher layers |
| '01' | The 2nd PUCCH resource value configured by the higher layers |
| '10' | The 3rd PUCCH resource value configured by the higher layers |
| '11' | The 4th PUCCH resource value configured by the higher layers |

- 10.1.2A	FDD-TDD HARQ-ACK feedback procedures for primary cell frame structure type 1

For a UE transmitting HARQ-ACK using PUCCH format 1b with channel selection, the UE shall determine the number of HARQ-ACK bits,  in subframe ,  based on the number of configured serving cells with subframe  configured as a downlink or special subframe according to the DL-reference UL/DL configuration (defined in subclause 10.2) of each serving cell and the downlink transmission modes configured for each serving cell. For a UE transmitting HARQ-ACK using PUCCH format 3, the UE shall determine the number of HARQ-ACK bits,  in subframe ,  based on the number of configured serving cells with subframe  configured as a downlink or special subframe except a special subframe of configurations 0 and 5 with normal downlink CP or of configurations 0 and 4 with extended downlink CP according to the DL-reference UL/DL configuration (defined in subclause 10.2) of each serving cell and the downlink transmission modes configured for each serving cell. The UE shall use two HARQ-ACK bits for a serving cell configured with a downlink transmission mode that support up to two transport blocks; and one HARQ-ACK bit otherwise.

A UE that supports aggregating at most 2 serving cells shall use PUCCH format 1b with channel selection for transmission of HARQ-ACK when configured with primary cell frame structure type 1 and secondary cell frame structure type 2.

A UE that supports aggregating more than 2 serving cells with primary cell frame structure type 1 is configured by higher layers to use either PUCCH format 1b with channel selection or PUCCH format 3 for transmission of HARQ-ACK when configured with more than one serving cell and primary cell frame structure type 1 and at least one secondary cell with frame structure type 2.

For HARQ-ACK transmission in subframe  with PUCCH format 1b with channel selection, the FDD-TDD HARQ-ACK procedure follows HARQ-ACK procedure described in subclause 10.1.2.1 if subframe  is an uplink or a special subframe of configurations 0 and 5 with normal downlink CP or of configurations 0 and 4 with extended downlink CP for the secondary cell according to the higher layer parameter subframeAssignment for UE not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, and according to the higher layer parameter eimta-HARQ-ReferenceConfig-r12 for UE configured with the higher layer parameter EIMTA-MainConfigServCell-r12, and HARQ-ACK procedure described in subclause 10.1.2.2.1 otherwise.

The FDD-TDD HARQ-ACK feedback procedure for PUCCH format 3 HARQ-ACK procedure as described in subclause 10.1.2.2.2.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 3.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1b with channel selection and with two configured serving cells.

### 10.1.3	TDD HARQ-ACK feedback procedures

For TDD and a UE that does not support aggregating more than one serving cell with frame structure type 2, two HARQ-ACK feedback modes are supported by higher layer configuration.

- HARQ-ACK bundling and

- HARQ-ACK multiplexing

For TDD UL/DL configuration 5 and a UE that does not support aggregating more than one serving cell with frame structure type 2 and the UE is not configured with EIMTA-MainConfigServCell-r12 for the serving cell, only HARQ-ACK bundling is supported.

A UE that supports aggregating more than one serving cell with frame structure type 2 is configured by higher layers to use either PUCCH format 1b with channel selection or PUCCH format 3 for transmission of HARQ-ACK when configured with more than one serving cell with frame structure type 2.

A UE that supports aggregating more than one serving cell with frame structure type 2 and is not configured with the parameter EIMTA-MainConfigServCell-r12 for any serving cell is configured by higher layers to use HARQ-ACK bundling, PUCCH format 1b with channel selection according to the set of Tables 10.1.3-2/3/4 or according to the set of Tables 10.1.3-5/6/7, or PUCCH format 3 for transmission of HARQ-ACK when configured with one serving cell with frame structure type 2.

A UE that is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell is configured by higher layers to use PUCCH format 1b with channel selection according to the set of Tables 10.1.3-5/6/7, or PUCCH format 3 for transmission of HARQ-ACK.

PUCCH format 1b with channel selection according to the set of Tables 10.1.3-2/3/4 or according to the set of Tables 10.1.3-5/6/7 is not supported for TDD UL/DL configuration 5.

TDD HARQ-ACK bundling is performed per codeword across  multiple downlink or special subframes associated with a single UL subframe n, where  is the number of elements in the set  defined in Table 10.1.3.1-1, by a logical AND operation of all the individual PDSCH transmission (with and without corresponding PDCCH/EPDCCH) HARQ-ACKs and ACK in response to PDCCH/EPDCCH indicating downlink SPS release. For one configured serving cell the bundled 1 or 2 HARQ-ACK bits are transmitted using PUCCH format 1a or PUCCH format 1b, respectively.

For TDD HARQ-ACK multiplexing and a subframe n with , where  is the number of elements in the set  defined in Table 10.1.3.1-1, spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is performed by a logical AND operation of all the corresponding individual HARQ-ACKs. PUCCH format 1b with channel selection is used in case of one configured serving cell. For TDD HARQ-ACK multiplexing and a subframe n with , spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is not performed, 1 or 2 HARQ-ACK bits are transmitted using PUCCH format 1a or PUCCH format 1b, respectively for one configured serving cell.

In the case of TDD and more than one configured serving cell with PUCCH format 1b with channel selection and more than 4 HARQ-ACK bits for  multiple downlink or special subframes associated with a single UL subframe n, where  is defined in subclause 10.1.3.2.1, and for the configured serving cells, spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe for all configured cells is performed and the bundled HARQ-ACK bits for each configured serving cell is transmitted using PUCCH format 1b with channel selection. For TDD and more than one configured serving cell with PUCCH format 1b with channel selection and up to 4 HARQ-ACK bits for multiple downlink or special subframes associated with a single UL subframe n, where  is defined in subclause 10.1.3.2.1, and for the configured serving cells, spatial HARQ-ACK bundling is not performed and the HARQ-ACK bits are transmitted using PUCCH format 1b with channel selection.

In the case of TDD and more than one configured serving cell with PUCCH format 3 and more than 20 HARQ-ACK bits for  multiple downlink or special subframes associated with a single UL subframe n, where  is the number of elements in the set  defined in subclause 10.1.3.2.2  and for the configured serving cells, spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is performed for each serving cell by a logical AND operation of all of the corresponding individual HARQ-ACKs and PUCCH format 3 is used. For TDD and more than one configured serving cell with PUCCH format 3 and up to 20 HARQ-ACK bits for  multiple downlink or special subframes associated with a single UL subframe n, where  is the number of elements in the set  defined in subclause 10.1.3.2.2 and for the configured serving cells, spatial HARQ-ACK bundling is not performed and the HARQ-ACK bits are transmitted using PUCCH format 3.

For TDD with PUCCH format 3, a UE shall determine the number of HARQ-ACK bits, , associated with an UL subframe n

according to   where  is the number of configured cells, and is the number of HARQ-bits for the c-th serving cell defined in subclause 7.3.

TDD HARQ-ACK feedback procedures for one configured serving cell are given in subclause 10.1.3.1 and procedures for more than one configured serving cell are given in subclause 10.1.3.2.

#### 10.1.3.1	TDD HARQ-ACK procedure for one configured serving cell

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1a/1b with TDD HARQ-ACK bundling feedback mode and for PUCCH format 3.

A UE that supports aggregating more than one serving cell with frame structure type 2 can be configured by higher layers for HARQ-ACK transmission on two antenna ports  for PUCCH format 1b with channel selection.

The TDD HARQ-ACK procedure for a UE configured with PUCCH format 3 is as described in subclause 10.1.3.2.2 when the UE receives PDSCH and/or SPS release PDCCH/EPDCCH only on the primary cell.

If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, for TDD HARQ-ACK bundling or TDD HARQ-ACK multiplexing for one configured serving cell and a subframe n with  where  is the number of elements in the set  defined in Table 10.1.3.1-1, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subframe  for  mapped to antenna port p for PUCCH format 1a/1b, where

- If there is PDSCH transmission indicated by the detection of corresponding PDCCH/EPDCCH or there is PDCCH/EPDCCH indicating downlink SPS release within subframe(s) , where  and  (defined in Table 10.1.3.1-1) is a set of M elements  depending on the subframe n and the UL/DL configuration (defined in Table 4.2-2 in [3]), and if PDCCH indicating PDSCH transmission or downlink SPS release is detected in subframe  , where  is the smallest value in set such that UE detects a PDCCH/EPDCCH indicating PDSCH transmission or downlink SPS release within subframe(s)  and , the UE first selects a  value out of {0, 1, 2, 3} which makes  and shall use  for antenna port , where  is configured by higher layers, , and  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe and the corresponding m. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for HARQ-ACK bundling for antenna port  is given by  .

- If there is only a PDSCH transmission where there is not a corresponding PDCCH/EPDCCH detected within subframe(s) , where and  is defined in Table 10.1.3.1-1, the UE shall use PUCCH format 1a/1b and PUCCH resource  with the value of  is determined according to higher layer configuration and Table 9.2-2.  For a UE configured for two antenna port transmission for PUCCH format 1a/1b and HARQ-ACK bundling, a PUCCH resource value in Table 9.2-2 maps to two PUCCH resources with the first PUCCH resource for antenna port  and the second PUCCH resource for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource for antenna port .

- If there is PDSCH transmission indicated by the detection of corresponding PDCCH/EPDCCH or there is PDCCH/EPDCCH indicating downlink SPS release within subframe(s) , where  and  (defined in Table 10.1.3.1-1) is a set of M elements  depending on the subframe n and the UL/DL configuration (defined in Table 4.2-2 in [3]), and if EPDCCH indicating PDSCH transmission or downlink SPS release is detected in subframe  , where  is the smallest value in set such that UE detects a PDCCH/EPDCCH indicating PDSCH transmission or downlink SPS release within subframe(s)  and , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

for antenna port , where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe  and the corresponding m,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3].  If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2.  If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe. If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for HARQ-ACK bundling for antenna port  is given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

Table 10.1.3.1-1: Downlink association set :  for TDD

| UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | - | 4 | - | - | 6 | - | 4 |
| 1 | - | - | 7, 6 | 4 | - | - | - | 7, 6 | 4 | - |
| 2 | - | - | 8, 7, 4, 6 | - | - | - | - | 8, 7, 4, 6 | - | - |
| 3 | - | - | 7, 6, 11 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 8, 7, 11 | 6, 5, 4, 7 | - | - | - | - | - | - |
| 5 | - | - | 13, 12, 9, 8, 7, 5, 4, 11, 6 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

Table 10.1.3.1-1A: eIMTA downlink association set : for TDD

| Higher layer parameter ‘eimta-HARQ-ReferenceConfig-r12’ | Higher layer parameter ‘subframeAssignment’ | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 2 | 0 | - | - | 7,8,4 | - | - | - | - | 7,8,4 | - | - |
|  | 1 | - | - | 8,4 | - | - | - | - | 8,4 | - | - |
|  | 6 | - | - | 6,8,4 | - | - | - | - | 8,6,4 | - | - |
| 4 | 0 | - | - | 12,7,11,8 | 7,4,5,6 | - | - | - | - | - | - |
|  | 1 | - | - | 12,8,11 | 7,5,6 | - | - | - | - | - | - |
|  | 3 | - | - | 12,8 | 4,7 | - | - | - | - | - | - |
|  | 6 | - | - | 12,11,8 | 4,5,6 | - | - | - | - | - | - |
| 5 | 0 | - | - | 12,7,11,13,8,4,9,5 | - | - | - | - | - | - | - |
|  | 1 | - | - | 13,12,8,11,4,9,5 | - | - | - | - | - | - | - |
|  | 2 | - | - | 13,12,9,11,5 | - | - | - | - | - | - | - |
|  | 3 | - | - | 13,12,5,4,8,9 | - | - | - | - | - | - | - |
|  | 4 | - | - | 13,5,4,6,9 | - | - | - | - | - | - | - |
|  | 6 | - | - | 13,12,11,6,8,4,9,5 | - | - | - | - | - | - | - |

Table 10.1.3.1-2: Mapping of ACK/NACK Resource offset Field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D to values for TDD when

| ACK/NACK Resource offset field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D |  |
| --- | --- |
| 0 | 0 |
| 1 |  |
| 2 |  |
| 3 | 2 |

Table 10.1.3.1-3: Mapping of ACK/NACK Resource offset Field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D to values for TDD when and

| ACK/NACK Resource offset field in DCI format 1A/1B/1D/1/2A/2/2B/2C/2D |  |
| --- | --- |
| 0 | 0 |
| 1 |  |
| 2 |  |
| 3 | 2 |

If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, for TDD HARQ-ACK multiplexing and sub-frame  with  and one configured serving cell, where  is the number of elements in the set  defined in Table 10.1.3.1-1, denote  as the PUCCH resource derived from sub-frame  and HARQ-ACK(i) as the ACK/NACK/DTX response from sub-frame , where (defined in Table 10.1.3.1-1) and .

- For a PDSCH transmission indicated by the detection of corresponding PDCCH or a PDCCH indicating downlink SPS release in sub-frame  where , the PUCCH resource , where  is selected from {0, 1, 2, 3} such that , ,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and  is configured by higher layers.

- For a PDSCH transmission where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2.

- For a PDSCH transmission indicated by the detection of corresponding EPDCCH or a EPDCCH indicating downlink SPS release in sub-frame  where , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1.  If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2, where the variable in the table is substituted with . If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0.

If a UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, then where the set is defined in Table 10.1.3.1-1 (where “UL/DL configuration” in the table refers to the higher layer parameter subframeAsssignment), and is the number of elements in set .

If a UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, then the set for the rest of this subclause is as defined in Sec 10.2, and is the number of elements for subframe n in the set

If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, for TDD HARQ-ACK multiplexing and sub-frame , denote  as the PUCCH resource derived from sub-frame  and HARQ-ACK(i0) as the ACK/NACK/DTX response from sub-frame, where , and .

-	 corresponding to each subframe , is determined as follows

Set ;

for

if the value of  is the same as the value of an element  in set , where ,

corresponding to subframe =;

end if

end for

for i3=0,1,...,MA−1

if the value of is same as the value of an element  in set , where  (defined in Table 10.1.3.1-1A)

corresponding to subframe =;

end if

end for

-	For a PDSCH transmission indicated by the detection of corresponding PDCCH or a PDCCH indicating downlink SPS release in sub-frame ,

-	if the value of is same as the value of an element in set , the PUCCH resource  is given by ;

-	otherwise, if the value of is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A), the UE shall set, the PUCCH resource  is given by nPUCCH,i0(1)=(MA−i3−1)⋅Nc+i3⋅Nc+1+nCCE,i+NPUCCHKA;

where  is the number of elements in the set  defined in Table 10.1.3.1-1A ,  is selected from {0, 1, 2, 3} such that , ,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers.

-	For a PDSCH transmission where there is not a corresponding PDCCH/EPDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2.

-	For a PDSCH transmission indicated by the detection of corresponding EPDCCH or a EPDCCH indicating downlink SPS release in sub-frame  where , the UE shall use

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of is same as the value of an index , where , then and ;

-	otherwise, if the value of is same as the value of an index , where , then and ;

, and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe  is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe  which is described in subclause 6.8A.5 in [3].

is determined as follows

-	If  and ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1.

-	If and ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2, where the variable in the table is substituted with, the variable in the table is substituted with , the variable  in the table is substituted with and the variable in the table is substituted with .

-	If and ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-3,

For a given subframe ,  is determined as follows

-	If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe .

-	If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe .

-	For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5,  is equal to 0.

-	For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7,  is equal to 0.

If a UE is not configured with two antenna port transmission for PUCCH format 1b with channel selection, and if the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, based on higher layer signalling the UE configured with a single serving cell will perform channel selection either according to the set of Tables 10.1.3-2, 10.1.3-3, and 10.1.3-4 or according to the set of Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7.

If a UE is configured with two antenna port transmission for PUCCH format 1b with channel selection, and if the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, then the UE will perform channel selection according to the set of Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7.

If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, the UE configured with a single serving cell will perform channel selection according to the set of Tables 10.1.3-5, 10.1.3-6, and 10.1.3-7.

For the selected table set, the UE shall transmit  on PUCCH resource in sub-frame  for  mapped to antenna port  using PUCCH format 1b according to subclause 5.4.1 in [3] where

-	=for antenna port and the value of  and the PUCCH resource  are generated by channel selection according to the selected set of Tables for M = 2, 3, and 4 respectively

-	  for antenna port, where  is selected from PUCCH resources configured by higher layers where , according to selected set of Tables for M = 2, 3, and 4 respectively by replacing with and replacing  with , when the UE is configured with two antenna port transmission for PUCCH format 1b with channel selection.

Table 10.1.3-2: Transmission of HARQ-ACK multiplexing for M = 2

| HARQ-ACK(0), HARQ-ACK(1) |  |  |
| --- | --- | --- |
| ACK, ACK |  | 1, 1 |
| ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK |  | 0, 0 |
| NACK/DTX, NACK |  | 1, 0 |
| NACK, DTX |  | 1, 0 |
| DTX, DTX | No transmission |  |

Table 10.1.3-3: Transmission of HARQ-ACK multiplexing for M = 3

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, NACK/DTX |  | 1, 1 |
| ACK, NACK/DTX, ACK |  | 1, 1 |
| ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| DTX, DTX, NACK |  | 0, 1 |
| DTX, NACK, NACK/DTX |  | 1, 0 |
| NACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| DTX, DTX, DTX | No transmission |  |

Table 10.1.3-4: Transmission of HARQ-ACK multiplexing for M = 4

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, ACK, NACK/DTX |  | 1, 0 |
| NACK/DTX,NACK/DTX,NACK,DTX |  | 1, 1 |
| ACK, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK, DTX, DTX, DTX |  | 1, 0 |
| ACK, ACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, NACK/DTX, NACK |  | 1, 1 |
| ACK, NACK/DTX, ACK, NACK/DTX |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, ACK |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK, DTX, DTX |  | 0, 0 |
| NACK/DTX, ACK, ACK, NACK/DTX |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| DTX, DTX, DTX, DTX | No transmission |  |

Table 10.1.3-5: Transmission of HARQ-ACK multiplexing for M = 2

| HARQ-ACK(0), HARQ-ACK(1) |  |  |
| --- | --- | --- |
| ACK, ACK |  | 1, 0 |
| ACK, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK |  | 0, 1 |
| NACK, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX | No Transmission |  |

Table 10.1.3-6: Transmission of HARQ-ACK multiplexing for M = 3

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK |  | 1, 0 |
| ACK, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| NACK, NACK/DTX, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX, NACK/DTX | No Transmission |  |

Table 10.1.3-7: Transmission of HARQ-ACK multiplexing for M = 4

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, ACK, NACK/DTX |  | 1, 1 |
| ACK, ACK, NACK/DTX, ACK |  | 1, 0 |
| ACK, ACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK, ACK |  | 1, 1 |
| ACK, NACK/DTX, ACK, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, NACK/DTX, ACK |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK, ACK |  | 0, 0 |
| NACK/DTX, ACK, ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| NACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX, NACK/DTX, NACK/DTX | No Transmission |  |

#### 10.1.3.2	TDD HARQ-ACK procedure for more than one configured serving cell

If a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, “UL/DL configuration” of the serving cell in the rest of this subclause refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

The TDD HARQ-ACK feedback procedures for more than one configured serving cell are either based on a PUCCH format 1b with channel selection HARQ-ACK procedure as described in subclause 10.1.3.2.1 or a PUCCH format 3 HARQ-ACK procedure as described in subclause 10.1.3.2.2.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 3 and TDD with more than one configured serving cell.

If a UE is configured with more than one serving cell and the TDD UL/DL configurations of all serving cells are the same, TDD UL/DL configuration 5 with PUCCH format 3 is only supported for up to two configured serving cells. 
If a UE is configured with two serving cells and the TDD UL/DL configuration of the two serving cells is the same, TDD UL/DL configuration 5 with PUCCH format 1b with channel selection for two configured serving cells is not supported. If a UE is configured with two serving cells and if the TDD UL/DL configuration of the two serving cells are not the same and if the DL-reference UL/DL configuration (as defined in subclause 10.2) of at least one serving cell is TDD UL/DL Configuration 5, PUCCH format 1b with channel selection is not supported.

If a UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, the UE is not expected to be configured with more than two serving cells having UL/DL Configuration 5 as a DL-reference UL/DL configuration.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1b with channel selection and TDD with two configured serving cells.

##### 10.1.3.2.1	PUCCH format 1b with channel selection HARQ-ACK procedure

If a UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, then where the set  is defined in Table 10.1.3.1-1 (where "UL/DL configuration" in the table refers to the higher layer parameter subframeAssignment), and is the number of elements in set .

If a UE is configured with two serving cells with the same UL/DL configurations, then in the rest of this subcaluse,  is as defined in Sec 10.2 and is the number of elements for subframe n in the set , and .

If a UE is configured with two serving cells with different UL/DL configurations,

-	then the UE shall determine  for a subframe n in this subclause as, where

-	 denotes the number of elements for subframe n in the set  for the primary cell (as defined in subclause 10.2)

-	 denotes the number of elements for subframe n in the set for the secondary serving cell (as defined in subclause 10.2)

-	if , then the UE shall, for the secondary serving cell, set HARQ-ACK(j) to DTX for j = to .

-	if , then the UE shall, for the primary cell, set HARQ-ACK(j) to DTX for j =  to

If the UE is configured with two serving cells with different UL/DL configurations, then in the rest of this subclause,  = where is defined in subclause 10.2.

For TDD HARQ-ACK multiplexing with PUCCH format 1b with channel selection and two configured serving cells and a subframe n with , a UE shall determine the number of HARQ-ACK bits, , based on the number of configured serving cells and the downlink transmission modes configured for each serving cell. The UE shall use two HARQ-ACK bits for a serving cell configured with a downlink transmission mode that supports up to two transport blocks; and one HARQ-ACK bit otherwise.

For TDD HARQ-ACK multiplexing with PUCCH format 1b with channel selection and two configured serving cells and a subframe n with , the UE shall transmit  on PUCCH resource  for  mapped to antenna port p using PUCCH format 1b where

-	 =  for antenna port , where  selected from  PUCCH resources,  where  and, according to Tables 10.1.3.2-1, 10.1.3.2-2, and 10.1.3.2-3 in subframe  using PUCCH format 1b.

-	  for antenna port, where  selected from  PUCCH resources, configured by higher layers where  and, according to Tables 10.1.3.2-1, 10.1.3.2-2, and 10.1.3.2-3 by replacing with and replacing  with  in subframe , when the UE is configured with two antenna port transmission for PUCCH format 1b with channel selection,

and for a subframe n with , HARQ-ACK(j) denotes the ACK/NACK/DTX response for a transport block or SPS release PDCCH/EPDCCH associated with serving cell, where the transport block and serving cell for HARQ-ACK(j) and  PUCCH resources are given by Table 10.1.2.2.1-1. For a subframe n with , HARQ-ACK(j) denotes the ACK/NACK/DTX response for a PDSCH transmission or SPS release PDCCH/EPDCCH within subframe(s) given by set  on each serving cell, where the subframes on each serving cell for HARQ-ACK(j) and  PUCCH resources are given by Table 10.1.3.2-4.

If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12, the UE shall determine the  PUCCH resources,  associated with HARQ-ACK(j) where  in Table 10.1.2.2.1-1 for  and Table 10.1.3.2-4 for , according to

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH in subframe , where  on the primary cell, or for a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where   on the primary cell, the PUCCH resource is , where  is selected from {0, 1, 2, 3} such that ,  where  is determined from the primary cell, and for a subframe n with  and a transmission mode that supports up to two transport blocks on the serving cell where the corresponding PDSCH transmission occurs, the PUCCH resource  is given by  where  is the number of the first CCE used for transmission of the corresponding DCI assignment and  is configured by higher layers.

- for a PDSCH transmission on the primary cell where there is not a corresponding PDCCH/EPDCCH detected within subframe(s) , where , the value of  is determined according to higher layer configuration and Table 9.2-2.

- For a PDSCH transmission indicated by the detection of corresponding EPDCCH or a EPDCCH indicating downlink SPS release in sub-frame  where on the primary cell, the PUCCH resource  is given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0.  For a subframe n with  and a transmission mode that supports up to two transport blocks on the serving cell where the corresponding PDSCH transmission occurs, the PUCCH resource  is given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH/EPDCCH within subframe(s) , where  on the secondary cell,  the value of , and the value of  for a subframe n with  or for a subframe n with  and a transmission mode on the secondary cell that supports up to two transport blocks is determined according to higher layer configuration and Table 10.1.2.2.1-2.  The TPC field in the DCI format of the corresponding PDCCH/EPDCCH shall be used to determine the PUCCH resource values from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.1-2.  For a UE configured for a transmission mode on the secondary cell that supports up to two transport blocks and a subframe n with , or for a subframe n with , a PUCCH resource value in Table 10.1.2.2.1-2 maps to two PUCCH resources , otherwise, the PUCCH resource value maps to a single PUCCH resource . A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted in the TPC field on all PDCCH/EPDCCH assignments on the secondary cell within subframe(s) , where .

Table 10.1.3.2-1: Transmission of HARQ-ACK multiplexing for A = 2

| HARQ-ACK(0), HARQ-ACK(1) |  |  |
| --- | --- | --- |
| ACK, ACK |  | 1, 0 |
| ACK, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK |  | 0, 1 |
| NACK, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX | No Transmission |  |

Table 10.1.3.2-2: Transmission of HARQ-ACK multiplexing for A = 3

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK |  | 1, 0 |
| ACK, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| NACK, NACK/DTX, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX, NACK/DTX | No Transmission |  |

Table 10.1.3.2-3: Transmission of HARQ-ACK multiplexing for A = 4

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, ACK, NACK/DTX |  | 1, 1 |
| ACK, ACK, NACK/DTX, ACK |  | 1, 0 |
| ACK, ACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK, ACK |  | 1, 1 |
| ACK, NACK/DTX, ACK, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, NACK/DTX, ACK |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK, ACK |  | 0, 0 |
| NACK/DTX, ACK, ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| NACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 0, 0 |
| DTX, NACK/DTX, NACK/DTX, NACK/DTX | No Transmission |  |

Table 10.1.3.2-4: Mapping of subframes on each serving cell to HARQ-ACK(j) for PUCCH format 1b HARQ-ACK channel selection for TDD with

| A | HARQ-ACK(j) |  |  |  |
| --- | --- | --- | --- | --- |
|  | HARQ-ACK(0) | HARQ-ACK(1) | HARQ-ACK(2) | HARQ-ACK(3) |
| 4 | The first subframe of Primary cell | The second subframe of Primary cell | The first subframe of Secondary cell | The second subframe of Secondary cell |

For TDD HARQ-ACK multiplexing with PUCCH format 1b with channel selection and sub-frame  with  and two configured serving cells, denotes  as the PUCCH resource derived from the transmissions in  downlink or special sub-frames associated with the UL subframe n  .  and are associated with the PDSCH transmission(s) or a PDCCH/EPDCCH indicating downlink SPS release (defined in subclause 9.2) on the primary cell and   and are associated with the PDSCH transmission(s) on the secondary cell.

For Primary cell:

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, and if there is a PDSCH transmission on the primary cell without a corresponding PDCCH/EPDCCH detected within the subframe(s) , where ,

-	the value of  is determined according to higher layer configuration and Table 9.2-2.

-	for a PDSCH transmission on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where  with the DAI value in the PDCCH equal to '1' (defined in Table 7.3-X) or a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the PDCCH equal to '1', the PUCCH resource  where  is selected from {0, 1, 2, 3} such that , , where  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe  and  is configured by higher layers.

-	for a PDSCH transmission on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where  with the DAI value in the EPDCCH equal to '1' (defined in Table 7.3-X) or an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the EPDCCH equal to '1', the PUCCH resource is given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe. For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0.

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, and if there is a PDSCH transmission on the primary cell without a corresponding PDCCH/EPDCCH detected within the subframe(s) , where ,

-	the value of  is determined according to higher layer configuration and Table 9.2-2.

-	for a PDSCH transmission on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where  with the DAI value in the PDCCH equal to '1' (defined in Table 7.3-X) or a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the PDCCH equal to '1',

-	if the value of is same as the value of an element , where , the PUCCH resource  is given by ;

-	otherwise, if the value of  is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A), the PUCCH resource  is given by ;

where  is the number of elements in the set  defined in Table 10.1.3.1-1A , where  is selected from {0, 1, 2, 3} such that ,  where  is determined from the primary cell,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers .

-	for a PDSCH transmission on the primary cell indicated by the detection of a corresponding EPDCCH in subframe, where  with the DAI value in the EPDCCH equal to '1' (defined in Table 7.3-X) or an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the EPDCCH equal to '1', the PUCCH resource is given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of  is same as the value of an index , where , then and ;

-	otherwise, if the value of is same as the value of an index , where , then and ;

, and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. , ,  are determined as described in section 10.1.3.1.

-	HARQ-ACK(0) is the ACK/NACK/DTX response for the PDSCH transmission without a corresponding PDCCH/EPDCCH.  For , if a PDSCH transmission with a corresponding PDCCH/EPDCCH and DAI value in the PDCCH/EPDCCH equal to '' or a PDCCH/EPDCCH indicating downlink SPS release and with DAI value in the PDCCH/EPDCCH equal to '' is received, HARQ-ACK(j) is the corresponding ACK/NACK/DTX response; otherwise  HARQ-ACK(j) shall be set to DTX.

-	Otherwise,

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, for a PDSCH transmission on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6} the DAI value in the PDCCH equal to either '1' or '2' or a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6} the DAI value in the PDCCH equal to either '1' or '2', the PUCCH resource , where  is selected from {0, 1, 2, 3} such that , , where  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe ,  is configured by higher layers and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6},  for the corresponding PDCCH with the DAI value equal to '1' and  for the corresponding PDCCH with the DAI value equal to '2', and for the primary cell with TDD UL/DL configuration 0  for the corresponding PDCCH.

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, for a PDSCH transmission on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6} the DAI value in the PDCCH equal to either '1' or '2' or a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6} the DAI value in the PDCCH equal to either '1' or '2',

-	if the value of is same as the value of an element , where , the PUCCH resource  is given by ;

-	otherwise, if the value of is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A), the PUCCH resource  is given by ;

where  is the number of elements in the set , where  is selected from {0, 1, 2, 3} such that ,  where  is determined from the primary cell,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers. Here, for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6},  for the corresponding PDCCH with the DAI value equal to '1' and  for the corresponding PDCCH with the DAI value equal to '2', and for the primary cell with TDD UL/DL configuration 0  for the corresponding PDCCH.

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell for a PDSCH transmission on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6} the DAI value in the EPDCCH equal to either  '1' or '2' or an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6}the DAI value in the EPDCCH equal to either '1' or '2', the PUCCH resource is given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0. Here, for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6}  for the corresponding EPDCCH with the DAI value equal to '1' and  for the corresponding EPDCCH with the DAI value equal to '2', and for the primary cell with TDD UL/DL configuration 0 for the corresponding EPDCCH.

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell for a PDSCH transmission on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6} the DAI value in the EPDCCH equal to either  '1' or '2' or an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe, where  and for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6}the DAI value in the EPDCCH equal to either '1' or '2', the PUCCH resource is given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of  is same as the value of an index , where , then ;

-	otherwise, if the value of  is same as the value of an index , where , then ;

, and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe  is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3].. , , are determined as described in section 10.1.3.1.  Here, for TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6}  for the corresponding EPDCCH with the DAI value equal to '1' and  for the corresponding EPDCCH with the DAI value equal to '2', and for the primary cell with TDD UL/DL configuration 0 for the corresponding EPDCCH.

-	For  and TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,6}, if a PDSCH transmission with a corresponding PDCCH/EPDCCH and DAI value in the PDCCH/EPDCCH equal to '' or a PDCCH/EPDCCH indicating downlink SPS release and with DAI value in the PDCCH/EPDCCH equal to '' is received, HARQ-ACK(j) is the corresponding ACK/NACK/DTX response; otherwise  HARQ-ACK(j) shall be set to DTX. For  and the primary cell with TDD UL/DL configuration 0, if a PDSCH transmission with a corresponding PDCCH/EPDCCH or a PDCCH/EPDCCH indicating downlink SPS release is received, HARQ-ACK(0) is the corresponding ACK/NACK/DTX response; otherwise HARQ-ACK(j) shall be set to DTX.

For Secondary cell:

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding PDCCH on the primary cell in subframe , where  with the DAI value in the PDCCH equal to either '1' or '2', the PUCCH resources , where  is selected from {0, 1, 2, 3} such that , , where  is determined from the primary cell,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe ,  is configured by higher layers,  for the corresponding PDCCH with the DAI value equal to '1' and  for the corresponding PDCCH with the DAI value equal to '2'.

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding PDCCH on the primary cell in subframe , where  with the DAI value in the PDCCH equal to either '1' or '2',

-	if the value of is same as the value of an element , where , the PUCCH resource  is given by ;

-	otherwise, if the value of is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A, where "UL/DL configuration" in the table refers to the higher layer parameter subframeAssignment), the PUCCH resource  is given by ;

where  is the number of elements in the set  defined in Table 10.1.3.1-1A , where  is selected from {0, 1, 2, 3} such that ,  where  is determined from the primary cell,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers. Here,  for the corresponding PDCCH with the DAI value equal to '1' and  for the corresponding PDCCH the DAI value in the PDCCH equal to either '1' or '2’.

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding EPDCCH on the primary cell in subframe , where  with the DAI value in the PDCCH equal to either '1' or '2', the PUCCH resources are given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe. For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0  or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0. Here,  for the corresponding EPDCCH with the DAI value equal to '1' and  for the corresponding EPDCCH with the DAI value equal to '2'.

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding EPDCCH on the primary cell in subframe , where  with the DAI value in the PDCCH equal to either '1' or '2', the PUCCH resources are given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of  is same as the value of an index , where , then ;

-	otherwise, if the value of  is same as the value of an index , where , then ;

and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe  is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3].  , , are determined as described in subclause 10.1.3.1. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7,  is equal to 0. Here,  for the corresponding EPDCCH with the DAI value equal to '1' and  for the corresponding EPDCCH with the DAI value equal to '2'.

-	for a PDSCH transmission indicated by the detection of a corresponding PDCCH/EPDCCH within the subframe(s) , where  on the secondary cell, the value of and  is determined according to higher layer configuration and Table 10.1.2.2.1-2. The TPC field in the DCI format of the corresponding PDCCH/EPDCCH shall be used to determine the PUCCH resource values from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.1-2. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted in the TPC field on all PDCCH/EPDCCH assignments on the secondary cell within subframe(s) , where .

-	For , if a PDSCH transmission with a corresponding PDCCH/EPDCCH and DAI value in the PDCCH/EPDCCH equal to '' is received, HARQ-ACK(j) is the corresponding ACK/NACK/DTX response; otherwise  HARQ-ACK(j) shall be set to DTX.

A UE shall perform channel selection according to the Tables 10.1.3.2-5, and 10.1.3.2-6 and transmit  on PUCCH resource  for  mapped to antenna port p using PUCCH format 1b according to subclause 5.4.1 in [3] where

-	 =  in sub-frame  for  mapped to antenna port  where "any" in Tables 10.1.3.2-5, and 10.1.3.2-6 represents any response of ACK, NACK, or DTX. The value of  and the PUCCH resource  are generated by channel selection according to Tables 10.1.3.2-5, and 10.1.3.2-6 for M = 3, and 4 respectively.

-	  for antenna port, where  selected from PUCCH resources, configured by higher layers where  according Tables 10.1.3.2-5, and 10.1.3.2-6 for M = 3, and 4 respectively by replacing with and replacing  with , where "any" in Tables 10.1.3.2-5, and 10.1.3.2-6 represents any response of ACK, NACK, or DTX, when the UE is configured with two antenna port transmission for PUCCH format 1b with channel selection.

Table 10.1.3.2-5: Transmission of HARQ-ACK multiplexing for M = 3

| Primary Cell | Secondary Cell | Resource | Constellation | RM Code Input Bits |
| --- | --- | --- | --- | --- |
| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) | HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |  |
| ACK, ACK, ACK | ACK, ACK, ACK |  | 1, 1 | 1,1,1,1 |
| ACK, ACK, NACK/DTX | ACK, ACK, ACK |  | 0, 0 | 1,0,1,1 |
| ACK, NACK/DTX, any | ACK, ACK, ACK |  | 1, 1 | 0,1,1,1 |
| NACK/DTX, any, any | ACK, ACK, ACK |  | 0, 1 | 0,0,1,1 |
| ACK, ACK, ACK | ACK, ACK, NACK/DTX |  | 1, 0 | 1,1,1,0 |
| ACK, ACK, NACK/DTX | ACK, ACK, NACK/DTX |  | 1, 0 | 1,0,1,0 |
| ACK, NACK/DTX, any | ACK, ACK, NACK/DTX |  | 0, 1 | 0,1,1,0 |
| NACK/DTX, any, any | ACK, ACK, NACK/DTX |  | 0, 0 | 0,0,1,0 |
| ACK, ACK, ACK | ACK, NACK/DTX, any |  | 1, 1 | 1, 1, 0, 1 |
| ACK, ACK, NACK/DTX | ACK, NACK/DTX, any |  | 0, 1 | 1, 0, 0, 1 |
| ACK, NACK/DTX, any | ACK, NACK/DTX, any |  | 1, 0 | 0, 1, 0, 1 |
| NACK/DTX, any, any | ACK, NACK/DTX, any |  | 0, 0 | 0, 0, 0, 1 |
| ACK, ACK, ACK | NACK/DTX, any, any |  | 1, 0 | 1, 1, 0, 0 |
| ACK, ACK, NACK/DTX | NACK/DTX, any, any |  | 0, 1 | 1, 0, 0, 0 |
| ACK, NACK/DTX, any | NACK/DTX, any, any |  | 1, 1 | 0, 1, 0, 0 |
| NACK, any, any | NACK/DTX, any, any |  | 0, 0 | 0, 0, 0, 0 |
| DTX, any, any | NACK/DTX, any, any | No Transmission |  | 0, 0, 0, 0 |

Table 10.1.3.2-6: Transmission of HARQ-ACK multiplexing for M = 4

| Primary Cell | Secondary Cell | Resource | Constellation | RM Code Input Bits |
| --- | --- | --- | --- | --- |
| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) | HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |  |
| ACK, ACK, ACK, NACK/DTX | ACK, ACK, ACK, NACK/DTX |  | 1, 1 | 1, 1, 1, 1 |
| ACK, ACK, NACK/DTX, any | ACK, ACK, ACK, NACK/DTX |  | 0, 0 | 1, 0, 1, 1 |
| ACK, DTX, DTX, DTX | ACK, ACK, ACK, NACK/DTX |  | 1, 1 | 0, 1, 1, 1 |
| ACK, ACK, ACK, ACK | ACK, ACK, ACK, NACK/DTX |  | 1, 1 | 0, 1, 1, 1 |
| NACK/DTX, any, any, any | ACK, ACK, ACK, NACK/DTX |  | 0, 1 | 0, 0, 1, 1 |
| (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | ACK, ACK, ACK, NACK/DTX |  | 0, 1 | 0, 0, 1, 1 |
| ACK, ACK, ACK, NACK/DTX | ACK, ACK, NACK/DTX, any |  | 1, 0 | 1, 1, 1, 0 |
| ACK, ACK, NACK/DTX, any | ACK, ACK, NACK/DTX, any |  | 1, 0 | 1, 0, 1, 0 |
| ACK, DTX, DTX, DTX | ACK, ACK, NACK/DTX, any |  | 0, 1 | 0, 1, 1, 0 |
| ACK, ACK, ACK, ACK | ACK, ACK, NACK/DTX, any |  | 0, 1 | 0, 1, 1, 0 |
| NACK/DTX, any, any, any | ACK, ACK, NACK/DTX, any |  | 0, 0 | 0, 0, 1, 0 |
| (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | ACK, ACK, NACK/DTX, any |  | 0, 0 | 0, 0, 1, 0 |
| ACK, ACK, ACK, NACK/DTX | ACK, DTX, DTX, DTX |  | 1, 1 | 1, 1, 0, 1 |
| ACK, ACK, ACK, NACK/DTX | ACK, ACK, ACK, ACK |  | 1, 1 | 1, 1, 0, 1 |
| ACK, ACK, NACK/DTX, any | ACK, DTX, DTX, DTX |  | 0, 1 | 1, 0, 0, 1 |
| ACK, ACK, NACK/DTX, any | ACK, ACK, ACK, ACK |  | 0, 1 | 1, 0, 0, 1 |
| ACK, DTX, DTX, DTX | ACK, DTX, DTX, DTX |  | 1, 0 | 0, 1, 0, 1 |
| ACK, DTX, DTX, DTX | ACK, ACK, ACK, ACK |  | 1, 0 | 0, 1, 0, 1 |
| ACK, ACK, ACK, ACK | ACK, DTX, DTX, DTX |  | 1, 0 | 0, 1, 0, 1 |
| ACK, ACK, ACK, ACK | ACK, ACK, ACK, ACK |  | 1, 0 | 0, 1, 0, 1 |
| NACK/DTX, any, any, any | ACK, DTX, DTX, DTX |  | 0, 0 | 0, 0, 0, 1 |
| NACK/DTX, any, any, any | ACK, ACK, ACK, ACK |  | 0, 0 | 0, 0, 0, 1 |
| (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | ACK, DTX, DTX, DTX |  | 0, 0 | 0, 0, 0, 1 |
| (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | ACK, ACK, ACK, ACK |  | 0, 0 | 0, 0, 0, 1 |
| ACK, ACK, ACK, NACK/DTX | NACK/DTX, any, any, any |  | 1, 0 | 1, 1, 0, 0 |
| ACK, ACK, ACK, NACK/DTX | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) |  | 1, 0 | 1, 1, 0, 0 |
| ACK, ACK, NACK/DTX, any | NACK/DTX, any, any, any |  | 0, 1 | 1, 0, 0, 0 |
| ACK, ACK, NACK/DTX, any | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) |  | 0, 1 | 1, 0, 0, 0 |
| ACK, DTX, DTX, DTX | NACK/DTX, any, any, any |  | 1, 1 | 0, 1, 0, 0 |
| ACK, DTX, DTX, DTX | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) |  | 1, 1 | 0, 1, 0, 0 |
| ACK, ACK, ACK, ACK | NACK/DTX, any, any, any |  | 1, 1 | 0, 1, 0, 0 |
| ACK, ACK, ACK, ACK | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) |  | 1, 1 | 0, 1, 0, 0 |
| NACK, any, any, any | NACK/DTX, any, any, any |  | 0, 0 | 0, 0, 0, 0 |
| NACK, any, any, any | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) |  | 0, 0 | 0, 0, 0, 0 |
| (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | NACK/DTX, any, any, any |  | 0, 0 | 0, 0, 0, 0 |
| (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) |  | 0, 0 | 0, 0, 0, 0 |
| DTX, any, any, any | NACK/DTX, any, any, any | No Transmission |  | 0, 0, 0, 0 |
| DTX, any, any, any | (ACK, NACK/DTX, any, any), except for (ACK, DTX, DTX, DTX) | No Transmission |  | 0, 0, 0, 0 |

##### 10.1.3.2.2	PUCCH format 3 HARQ-ACK procedure

If a UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12, then where the set  is defined in Table 10.1.3.1-1 (where "UL/DL configuration" in the table refers to the higher layer parameter subframeAssignment), and is the number of elements in set .

If a UE is configured with one serving cell, or if a UE is configured with more than one serving cells and the UL/DL configuration of all serving cells is same, then in the rest of this subclause  is as defined in Sec 10.2, and  is the number of elements in the set .

If a UE is configured with more than one serving cell and if at least two cells have different UL/DL configurations, then in this subclause refers to  (as defined in subclause 10.2) , and  is the number of elements in the set .

For TDD HARQ-ACK transmission with PUCCH format 3  and sub-frame  with  and more than one configured serving cell, where  is the number of elements in the set , the UE shall use PUCCH resource  or  for transmission of HARQ-ACK in subframe  for  mapped to antenna port p where

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell,

-	for a single PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the PDCCH is equal to '1' (defined in Table 7.3-X), or

-	for a single PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the PDCCH is equal to '1',

-	the UE shall use PUCCH format 1a/1b and PUCCH resource  with  for antenna port , where  is configured by higher layers,  is selected from {0, 1, 2, 3} such that , , and  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe  where . When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for antenna port  is given by

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell,

-	for a single PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the PDCCH is equal to '1' (defined in Table 7.3-X), or

-	for a single PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the PDCCH is equal to '1',

-	the UE shall use PUCCH format 1a/1b, and

-	if the value of is same as the value of an element , where , the PUCCH resource  is given by ;

-	otherwise, if the value of is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A, where "UL/DL configuration" in the table refers to the higher layer parameter subframeAssignment), the PUCCH resource  is given by ;

where  is the number of elements in the set  defined in Table 10.1.3.1-1A , where  is selected from {0, 1, 2, 3} such that ,  where  is determined from the primary cell,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for antenna port  is given by

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell,

-	for a single PDSCH transmission only on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the EPDCCH is equal to '1' (defined in Table 7.3-X), or

-	for a single PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the EPDCCH is equal to '1',

-	the UE shall use PUCCH format 1a/1b and PUCCH resource  given by

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe . For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for antenna port  is given by .

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell,

-	for a single PDSCH transmission only on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the EPDCCH is equal to '1' (defined in Table 7.3-X), or

-	for a single PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where , and for a TDD UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} the DAI value in the EPDCCH is equal to '1',

-	the UE shall use PUCCH format 1a/1b and PUCCH resource  given by

-	if EPDCCH-PRB-set is configured for distributed transmission

-	if EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of  is same as the value of an index , where , then ;

-	otherwise, if the value of  is same as the value of an index , where , then ;

and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe  is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3].  , , are determined as described in section 10.1.3.1. When two antenna port transmission is configured for PUCCH format 1a/1b, the PUCCH resource for antenna port  is given by .

-	for a single PDSCH transmission only on the primary cell where there is not a corresponding PDCCH/EPDCCH detected within subframe(s) , where  and no PDCCH/EPDCCH indicating downlink SPS release (defined in subclause 9.2) within subframe(s) , where , the UE shall use PUCCH format 1a/1b and PUCCH resource  with the value of  is determined according to higher layer configuration and Table 9.2-2. For a UE configured for two antenna port transmission for PUCCH format 1a/1b, a PUCCH resource value in Table 9.2-2 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

-	for , and

-	for a PDSCH transmission only on the primary cell where there is not a corresponding PDCCH detected within subframe(s) , where , and

-	for an additional PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where  with the DAI value in the PDCCH equal to '1' (defined in Table 7.3-X), or

-	for an additional PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the PDCCH equal to '1',

-	the UE shall transmit  in subframe  using PUCCH format 1b on PUCCH resource  selected from  PUCCH resources  where , according to Table 10.1.3.2-1 and Table 10.1.3.2-2 for  and , respectively. For a UE configured with a transmission mode that supports up to two transport blocks on the primary cell, ; otherwise, .

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, the PUCCH resource  is determined according to higher layer configuration and Table 9.2-2. The PUCCH resource  is determined as , where  is configured by higher layers,  is selected from {0, 1, 2, 3} such that , , and  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe  where .

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, the PUCCH resource  is determined according to higher layer configuration and Table 9.2-2. The PUCCH resource  is determined as

-	if the value of is same as the value of an element , where , the PUCCH resource is given by ;

-	otherwise, if the value of is same as the value of an element  in set , where (defined in Table 10.1.3.1-1A, where "UL/DL configuration" in the table refers to the higher layer parameter subframeAssignment), the PUCCH resource  is given by ;

where  is the number of elements in the set  defined in Table 10.1.3.1-1A , where  is selected from {0, 1, 2, 3} such that , ,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and , , are configured by higher layers.

-	For a UE configured with a transmission mode that supports up to two transport blocks on the primary cell, the PUCCH resource  is determined as .HARQ-ACK(0) is the ACK/NACK/DTX response for the PDSCH without a corresponding PDCCH detected. HARQ-ACK(1) is the ACK/NACK/DTX response for the first transport block of the PDSCH indicated by the detection of a corresponding PDCCH for which the value of the DAI field in the corresponding DCI format is equal to '1' or for the PDCCH indicating downlink SPS release for which the value of the DAI field in the corresponding DCI format is equal to '1'. HARQ-ACK(2) is the ACK/NACK/DTX response for the second transport block of the PDSCH indicated by the detection of a corresponding PDCCH for which the value of the DAI field in the corresponding DCI format is equal to '1'.

-	for , and

-	for a PDSCH transmission only on the primary cell where there is not a corresponding EPDCCH detected within subframe(s) , where  , and

-	for an additional PDSCH transmission only on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where  with the DAI value in the EPDCCH equal to '1' (defined in Table 7.3-X), or

-	for an additional EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the EPDCCH equal to '1',

-	the UE shall transmit  in subframe  using PUCCH format 1b on PUCCH resource  selected from  PUCCH resources  where , according to Table 10.1.3.2-1 and Table 10.1.3.2-2 for  and , respectively. For a UE configured with a transmission mode that supports up to two transport blocks on the primary cell, ; otherwise, .

-	If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, the PUCCH resource  is determined according to higher layer configuration and Table 9.2-2. The PUCCH resource  is determined as

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3]. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.2.1-1. If ,  is determined from the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH as given in Table 10.1.3.1-2. If the UE is configured to monitor EPDCCH in subframe ,  is equal to the number of ECCEs in EPDCCH-PRB-set configured for that UE in subframe . If the UE is not configured to monitor EPDCCH in subframe,  is equal to the number of ECCEs computed assuming EPDCCH-PRB-set is configured for that UE in subframe. For normal downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 5, is equal to 0. For extended downlink CP, if subframe  is a special subframe with special subframe configuration 0 or 4 or 7, is equal to 0.

-	If the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 on the primary cell, the PUCCH resource  is determined according to higher layer configuration and Table 9.2-2. The PUCCH resource  is determined as

-	If EPDCCH-PRB-set is configured for distributed transmission

-	If EPDCCH-PRB-set is configured for localized transmission

where

-	if the value of  is same as the value of an index , where , then ;

-	otherwise, if the value of  is same as the value of an index , where , then ;

-	and where  is the number of the first ECCE (i.e. lowest ECCE index used to construct the EPDCCH) used for transmission of the corresponding DCI assignment in EPDCCH-PRB-set in subframe ,  for EPDCCH-PRB-set is configured by the higher layer parameter pucch-ResourceStartOffset-r11 , for EPDCCH-PRB-set in subframe  is given in subclause 6.8A.1 in [3], is determined from the antenna port used for EPDCCH transmission in subframe which is described in subclause 6.8A.5 in [3].  , , are determined as described in section 10.1.3.1.

-	For a UE configured with a transmission mode that supports up to two transport blocks on the primary cell, the PUCCH resource  is determined as .HARQ-ACK(0) is the ACK/NACK/DTX response for the PDSCH without a corresponding EPDCCH detected. HARQ-ACK(1) is the ACK/NACK/DTX response for the first transport block of the PDSCH indicated by the detection of a corresponding EPDCCH for which the value of the DAI field in the corresponding DCI format is equal to '1' or for the EPDCCH indicating downlink SPS release for which the value of the DAI field in the corresponding DCI format is equal to '1'. HARQ-ACK(2) is the ACK/NACK/DTX response for the second transport block of the PDSCH indicated by the detection of a corresponding EPDCCH for which the value of the DAI field in the corresponding DCI format is equal to '1'.

-	for , and

-	for a PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH in subframe , where  with the DAI value in the PDCCH greater than '1' (defined in Table 7.3-X), or

-	for a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the PDCCH greater than '1', or

-	for M=9 and for a PDSCH transmission only on the primary cell indicated by the detection of a corresponding PDCCH in subframe n−km, where km∈K with the DAI value in the PDCCH equal to '1' (defined in Table 7.3-X) not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1', or

-	for M=9 and for a PDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe n−km, where km∈K with the DAI value in the PDCCH equal to ‘1’ (defined in Table 7.3-X) not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1',

-	the UE shall use PUCCH format 3 and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 10.1.2.2.2-1 and the TPC field in a PDCCH assignment with DAI value greater than '1' (defined in Table 7.3-X) or with DAI value equal to ‘1’, not being the first PDCCH/EPDCCH assignment in subframe(s) n−k, where k∈K with the DAI value equal to '1', shall be used to determine the PUCCH resource value from one of the four PUCCH resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted on all PDCCH assignments used to determine the PUCCH resource values within the subframe(s) , where .

-	for , and

-	for a PDSCH transmission only on the primary cell indicated by the detection of a corresponding EPDCCH in subframe , where  with the DAI value in the EPDCCH greater than '1' (defined in Table 7.3-X), or

-	for an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe , where  with the DAI value in the EPDCCH greater than '1', or

-	for M=9 and for a PDSCH transmission only on the primary cell indicated by the detection of a corresponding EPDCCH in subframe n−km, where km∈K with the DAI value in the EPDCCH equal to '1' (defined in Table 7.3-X) not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1', or

-	for M=9 and for an EPDCCH indicating downlink SPS release (defined in subclause 9.2) in subframe n−km, where km∈K with the DAI value in the EPDCCH equal to ‘1’ (defined in Table 7.3-X) not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1',

-	the UE shall use PUCCH format 3 and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 10.1.2.2.2-1 and the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH assignment with DAI value greater than '1' or with DAI value equal to ‘1’ (defined in Table 7.3-X), not being the first PDCCH/EPDCCH assignment in subframe(s) n−k, where k∈K with the DAI value equal to '1', shall be used to determine the PUCCH resource value from one of the four PUCCH resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted on all EPDCCH assignments used to determine the PUCCH resource values within the subframe(s) , where .

-	If the UL/DL configurations of all serving cells are the same, for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding PDCCH/EPDCCH within subframe(s) , where , the UE shall use PUCCH format 3 and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 10.1.2.2.2-1 and the TPC field in the corresponding PDCCH/EPDCCH shall be used to determine the PUCCH resource value from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. For TDD UL/DL configurations 1-6, if a PDCCH corresponding to a PDSCH on the primary cell within subframe(s) , where , or a PDCCH indicating downlink SPS release (defined in subclause 9.2) within subframe(s) , where , is detected, the TPC field in the PDCCH with the DAI value greater than '1' or with DAI value equal to ‘1’, not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1' (defined in Table 7.3-X), shall be used to determine the PUCCH resource value from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted on all PDCCH assignments in the primary cell and in each secondary cell that are used to determined the PUCCH resource value within the subframe(s) , where . For TDD UL/DL configurations 1-6, if an EPDCCH corresponding to a PDSCH on the primary cell within subframe(s) , where , or an EPDCCH indicating downlink SPS release (defined in subclause 9.2) within subframe(s) , where , is detected, the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH assignment with the DAI value greater than '1' (defined in Table 7.3-X) or with DAI value equal to ‘1’, not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1', shall be used to determine the PUCCH resource value from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted on all EPDCCH assignments in the primary cell and in each secondary cell that are used to determined the PUCCH resource value within the subframe(s) , where .

-	If the UL/DL configurations of at least two serving cells are different, for a PDSCH transmission on the secondary cell indicated by the detection of a corresponding PDCCH/EPDCCH within subframe(s) , where , the UE shall use PUCCH format 3 and PUCCH resource  where the value of  is determined according to higher layer configuration and Table 10.1.2.2.2-1 and the TPC field in the corresponding PDCCH/EPDCCH shall be used to determine the PUCCH resource value from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. For a UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} as defined in subclause 10.2, if a PDCCH corresponding to a PDSCH on the primary cell within subframe(s) , where , or a PDCCH indicating downlink SPS release (defined in subclause 9.2) within subframe(s) , where , is detected, the TPC field in the PDCCH with the DAI value greater than '1' (defined in Table 7.3-X) or with DAI value equal to ‘1’, not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1', shall be used to determine the PUCCH resource value from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted on all PDCCH assignments in the primary cell and in each secondary cell that are used to determined the PUCCH resource value within the subframe(s) , where . For a UL/DL configuration of the primary cell belonging to {1,2,3,4,5,6} as defined in subclause 10.2, if an EPDCCH corresponding to a PDSCH on the primary cell within subframe(s) , where , or an EPDCCH indicating downlink SPS release (defined in subclause 9.2) within subframe(s) , where , is detected, the HARQ-ACK resource offset field in the DCI format of the corresponding EPDCCH assignment with the DAI value greater than '1' or with DAI value equal to ‘1’, not being the first PDCCH/EPDCCH transmission in subframe(s) n−k, where k∈K with the DAI value equal to '1', shall be used to determine the PUCCH resource value from one of the four resource values configured by higher layers, with the mapping defined in Table 10.1.2.2.2-1. A UE shall assume that the same HARQ-ACK PUCCH resource value is transmitted on all EPDCCH assignments in the primary cell and in each secondary cell that are used to determined the PUCCH resource value within the subframe(s) , where .

-	For PUCCH format 3 and PUCCH resource  and a UE configured for two antenna port transmission, a PUCCH resource value in Table 10.1.2.2.2-1 maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource  for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

### 10.1.3A	FDD-TDD HARQ-ACK feedback procedures for primary cell frame structure type 2

A UE is configured by higher layers to use either PUCCH format 1b with channel selection or PUCCH format 3 for transmission of HARQ-ACK.

For a serving cell, if the serving cell is frame structure type 1, and a UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, set is defined in Table 10.1.3A-1, otherwise set is defined in Table 10.1.3.1-1.

PUCCH format 1b with channel selection is not supported if a UE is configured with more than two serving cells, or if the DL-reference UL/DL configuration 5 (as defined in subclause 10.2) is defined for any serving cell, or if the DL-reference UL/DL configuration of a serving cell with frame structure type 1 belongs to {2, 3, 4} and the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell.

If a UE is configured with the parameter EIMTA-MainConfigServCell-r12 for at least one serving cell, the UE is not expected to be configured with more than two serving cells having DL-reference UL/DL configuration 5.

If a UE is configured to use PUCCH format 1b with channel selection for HARQ-ACK transmission, for the serving cells,

-	if  more than 4 HARQ-ACK bits for multiple downlink and special subframes associated with a single UL subframe n, where  is as defined in subclause 10.1.3.2.1 for case where the UE is configured with two serving cells with different UL/DL configurations,

-	spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is performed for each serving cell by a logical AND operation of all the corresponding individual HARQ-ACKs, and the bundled HARQ-ACK bits for each serving cell is transmitted using PUCCH format 1b with channel selection,

-	otherwise,

-	spatial HARQ-ACK bundling is not performed, and the HARQ-ACK bits are transmitted using PUCCH format 1b with channel selection.

If a UE is configured to use PUCCH format 3 for HARQ-ACK transmission, for the serving cells,

-	if  more than 21 HARQ-ACK bits for  multiple downlink and special subframes associated with a single UL subframe n, where  as defined in subclause 10.1.3.2.2 for the case of UE configured with more than one serving cell and if at least two cells have different UL/DL configurations,

-	spatial HARQ-ACK bundling across multiple codewords within a downlink or special subframe is performed for each serving cell by a logical AND operation of all of the corresponding individual HARQ-ACKs, and PUCCH format 3 is used,

-	otherwise,

-	spatial HARQ-ACK bundling is not performed, and the HARQ-ACK bits are transmitted using PUCCH format 3.

-	UE shall determine the number of HARQ-ACK bits, ,  associated with an UL subframe n according to   where  is the number of configured cells, and is the number of HARQ-bits for the c-th serving cell defined in subclause 7.3.4. If a UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling a serving cell with frame structure type 1, and the DL-reference UL/DL configuration of the serving cell belongs to {2, 3, 4, 5}, then the UE is not expected to be configured with  which result in .

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 3.

HARQ-ACK transmission on two antenna ports  is supported for PUCCH format 1b with channel selection and with two configured serving cells.

The FDD-TDD HARQ-ACK feedback procedure for PUCCH format 1b with channel selection follows the HARQ-ACK procedure described in subclause 10.1.3.2.1 for the case of UE configured with two serving cells with different UL/DL configurations, and for PUCCH format 3 follows the HARQ-ACK procedure described in subclause 10.1.3.2.2 for the case of UE configured with more than one serving cell and if at least two cells have different UL/DL configurations.

Table 10.1.3A-1: Downlink association set :  for FDD-TDD and serving cell frame structure type 1

| DL-reference UL/DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6, 5 | 5, 4 | 4 | - | - | 6, 5 | 5, 4 | 4 |
| 1 | - | - | 7, 6 | 6, 5, 4 | - | - | - | 7, 6 | 6, 5, 4 | - |
| 2 | - | - | 8, 7, 6, 5, 4 | - | - | - | - | 8, 7, 6, 5, 4 | - | - |
| 3 | - | - | 11, 10, 9, 8, 7, 6 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 11, 10, 9, 8, 7 | 7, 6, 5, 4 | - | - | - | - | - | - |
| 5 | - | - | 13, 12, 11, 10, 9, 8, 7, 6, 5, 4 | - | - | - | - | - | - | - |
| 6 | - | - | 8, 7 | 7, 6 | 6, 5 | - | - | 7 | 7, 6, 5 | - |

### 10.1.4	HARQ-ACK Repetition procedure

HARQ-ACK repetition is enabled or disabled by a UE specific parameter ackNackRepetition configured by higher layers. Once enabled, the UE shall repeat any HARQ-ACK transmission with a repetition factor , where  is provided by higher layers and includes the initial HARQ-ACK transmission, until HARQ-ACK repetition is disabled by higher layers. For a PDSCH transmission without a corresponding PDCCH/EPDCCH detected, the UE shall transmit the corresponding HARQ-ACK response  times using PUCCH resource  configured by higher layers. For a PDSCH transmission with a corresponding PDCCH/EPDCCH detected, or for a PDCCH/EPDCCH indicating downlink SPS release, the UE shall first transmit the corresponding HARQ-ACK response once using PUCCH resource derived from the corresponding PDCCH CCE index or EPDCCH ECCE index (as described in subclauses 10.1.2 and 10.1.3), and repeat the transmission of the corresponding HARQ-ACK response   times always using PUCCH resource , where is configured by higher layers.

HARQ-ACK repetition is only applicable for UEs configured with one serving cell for FDD and TDD. For TDD, HARQ-ACK repetition is only applicable for HARQ-ACK bundling.

HARQ-ACK repetition can be enabled with PUCCH format 1a/1b on two antenna ports. For a UE configured for two antenna port transmission for HARQ-ACK repetition with PUCCH format 1a/1b, a PUCCH resource value  maps to two PUCCH resources with the first PUCCH resource  for antenna port  and the second PUCCH resource for antenna port , otherwise, the PUCCH resource value maps to a single PUCCH resource  for antenna port .

### 10.1.5	Scheduling Request (SR) procedure

A UE is configured by higher layers to transmit the SR on one antenna port or two antenna ports. 
The scheduling request shall be transmitted on the PUCCH resource(s) for  mapped to antenna port p as defined in [3], where  is configured by higher layers unless the SR coincides in time with the transmission of HARQ-ACK using PUCCH Format 3 in which case the SR is multiplexed with HARQ-ACK according to subclause 5.2.3.1 of [4]. The SR configuration for SR transmission periodicity and SR subframe offset  is defined in Table 10.1.5-1 by the parameter sr-ConfigIndex  given by higher layers.

SR transmission instances are the uplink subframes satisfying .

Table 10.1.5-1: UE-specific SR periodicity and subframe offset configuration

| SR configuration Index | SR periodicity (ms) | SR subframe offset |
| --- | --- | --- |
| 0 – 4 | 5 |  |
| 5 – 14 | 10 |  |
| 15 – 34 | 20 |  |
| 35 – 74 | 40 |  |
| 75 – 154 | 80 |  |
| 155 – 156 | 2 |  |
| 157 | 1 |  |

## 10.2	Uplink HARQ-ACK timing

For TDD or for FDD-TDD and primary cell frame structure type 2 or for FDD-TDD and primary cell frame structure type 1, if a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell, “UL/DL configuration” of the serving cell in subclause 10.2 refers to the UL/DL configuration given by the parameter eimta-HARQ-ReferenceConfig-r12 for the serving cell unless specified otherwise.

For FDD or for FDD-TDD and primary cell frame structure type 1, the UE shall upon detection of a PDSCH transmission in subframe n-4 intended for the UE and for which an HARQ-ACK shall be provided, transmit the HARQ-ACK response in subframe n. If HARQ-ACK repetition is enabled, upon detection of a PDSCH transmission in subframe n-4 intended for the UE and for which HARQ-ACK response shall be provided, and if the UE is not repeating the transmission of any HARQ-ACK in subframe  corresponding to a PDSCH transmission in subframes , … , , the UE:

-	shall transmit only the HARQ-ACK response (corresponding to the detected PDSCH transmission in subframe ) on PUCCH in subframes , , …, ;

-	shall not transmit any other signal/channel in subframes , , …, ; and

-	shall not transmit any HARQ-ACK response repetitions corresponding to any detected PDSCH transmission in subframes , …, .

For TDD and a UE configured with EIMTA-MainConfigServCell-r12 for at least one serving cell, if the UE is configured with one serving cell or if the UE is configured with more than one serving cell and the TDD UL/DL configuration of all the configured serving cells is the same, the DL-reference UL/DL configuration for a serving cell is the UL/DL configuration of the serving cell.

For FDD-TDD and primary cell frame structure type 1, if a serving cell is a secondary serving cell with frame structure type 2, the DL-reference UL/DL configuration for the serving cell is the UL/DL configuration of the serving cell.

For TDD, if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations and if a serving cell is a primary cell, then the primary cell UL/DL configuration is the DL-reference UL/DL configuration for the serving cell.

For FDD-TDD and primary cell frame structure type 2, if a serving cell is a primary cell or if a serving cell is a secondary cell with frame structure type 1, then the primary cell UL/DL configuration is the DL-reference UL/DL configuration for the serving cell.

For TDD and if the UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations and if a serving cell is a secondary cell, or for FDD-TDD and primary cell frame structure type 2 and if a serving cell is a secondary cell with frame structure type 2

-	if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration ) belongs to Set 1 in Table 10.2-1 or

-	if the UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, and if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration ) belongs to Set 2 or Set 3 in Table 10.2-1 or

-	if the UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell, and if the pair formed by (primary cell UL/DL configuration, serving cell UL/DL configuration) belongs to Set 4 or Set 5 in Table 10.2-1

then the DL-reference UL/DL configuration for the serving cell is defined in the corresponding Set in Table 10.2-1.

For TDD and if a UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations or for FDD-TDD and primary cell frame structure type 2, if the DL-reference UL/DL configuration for at least one serving cell is TDD UL/DL Configuration 5, then the UE is not expected to be configured with more than two serving cells.

For TDD and a UE not configured with EIMTA-MainConfigServCell-r12 for any serving cell, if the UE is configured with one serving cell, or the UE is configured with more than one serving cell and the UL/DL configurations of all serving cells is same, then the UE shall upon detection of a PDSCH transmission within subframe(s) , where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe n.

For TDD and if a UE is configured with more than one serving cell and if at least two serving cells have different UL/DL configurations, or if a UE is configured with EIMTA-MainConfigServCell-r12 for at least one serving cell. or for FDD-TDD and primary cell frame structure type 2 and if a serving cell c is frame structure type 2, then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where  intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in UL subframe n, wherein set contains values of such that subframe n-k corresponds to a DL subframe or a special subframe for serving cell , where DL subframe or special subframe of serving cell  is according to the higher layer parameter eimta-HARQ-ReferenceConfig-r12 if the UE is configured with the higher layer parameter EIMTA-MainConfigServCell-r12 for  serving cell ;  defined in Table 10.1.3.1-1 (where "UL/DL configuration" in Table 10.1.3.1-1 refers to the "DL-reference UL/DL configuration") is associated with subframe n.

For FDD-TDD and primary cell frame structure type 2, if a serving cell  is frame structure type 1 and a UE is not configured to monitor PDCCH/EPDCCH in another serving cell for scheduling the serving cell , then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where , and is defined in Table 10.1.3A-1 intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in subframe n.

For FDD-TDD and primary cell frame structure type 2, if a serving cell  is frame structure type 1 and a UE is configured to monitor PDCCH/EPDCCH in another serving cell for scheduling serving cell , then the UE shall upon detection of a PDSCH transmission within subframe(s)  for serving cell , where , and is defined in Table 10.1.3.1-1, intended for the UE and for which HARQ-ACK response shall be provided, transmit the HARQ-ACK response in subframe n, where "UL/DL configuration" in Table 10.1.3.1-1 refers to the "DL-reference UL/DL configuration" of serving cell .

For TDD, if HARQ-ACK repetition is enabled, upon detection of a PDSCH transmission within subframe(s) , where  and  is defined in Table 10.1.3.1-1 intended for the UE and for which HARQ-ACK response shall be provided, and if the UE is not repeating the transmission of any HARQ-ACK in subframe  corresponding to a PDSCH transmission in a downlink or special subframe earlier than subframe , the UE:

-	shall transmit only the HARQ-ACK response (corresponding to the detected PDSCH transmission in subframe) on PUCCH in UL subframe and the next UL subframes denoted as , …,;

-	shall not transmit any other signal/channel in UL subframe , , …,; and

-	shall not transmit any HARQ-ACK response repetitions corresponding to any detected PDSCH transmission in subframes  , where , is the set defined in Table 10.1.3.1-1 corresponding to UL subframe , and .

For TDD, HARQ-ACK bundling, if the UE detects that at least one downlink assignment has been missed as described in subclause 7.3, the UE shall not transmit HARQ-ACK on PUCCH if HARQ-ACK is the only UCI present in a given subframe.

The uplink timing for the ACK corresponding to a detected PDCCH/EPDCCH indicating downlink SPS release shall be the same as the uplink timing for the HARQ-ACK corresponding to a detected PDSCH, as defined above.

Table 10.2-1: DL-reference UL/DL configuration for serving cell based on pair formed by (primary cell UL/DL configuration, secondary cell UL/DL configuration)

| Set # | (Primary cell UL/DL configuration, Secondary cell UL/DL configuration) | DL-reference UL/DL configuration |
| --- | --- | --- |
| Set 1 | (0,0) | 0 |
|  | (1,0),(1,1),(1,6) | 1 |
|  | (2,0),(2,2),(2,1),(2,6) | 2 |
|  | (3,0),(3,3),(3,6) | 3 |
|  | (4,0),(4,1),(4,3),(4,4),(4,6) | 4 |
|  | (5,0),(5,1),(5,2),(5,3),(5,4),(5,5),(5,6) | 5 |
|  | (6,0),(6,6) | 6 |
| Set 2 | (0,1),(6,1) | 1 |
|  | (0,2),(1,2),(6,2) | 2 |
|  | (0,3),(6,3) | 3 |
|  | (0,4),(1,4),(3,4),(6,4) | 4 |
|  | (0,5),(1,5),(2,5),(3,5),(4,5),(6,5) | 5 |
|  | (0,6) | 6 |
| Set 3 | (3,1),(1,3) | 4 |
|  | (3,2),(4,2),(2,3),(2,4) | 5 |
| Set 4 | (0,1),(0,2),(0,3),(0,4),(0,5),(0,6) | 0 |
|  | (1,2),(1,4),(1,5) | 1 |
|  | (2,5) | 2 |
|  | (3,4),(3,5) | 3 |
|  | (4,5) | 4 |
|  | (6,1),(6,2),(6,3),(6,4),(6,5) | 6 |
| Set 5 | (1,3) | 1 |
|  | (2,3),(2,4) | 2 |
|  | (3,1),(3,2) | 3 |
|  | (4,2) | 4 |

# 11	Physical Multicast Channel (PMCH) related procedures

## 11.1	UE procedure for receiving the PMCH

The UE shall decode the PMCH when configured by higher layers. The UE may assume that an eNB transmission on the PMCH is performed according to subclause 6.5 of [3].

Thefor the PMCH is configured by higher layers. If the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM then the UE shall useand Table 7.1.7.1-1A to determine the modulation order () and TBS index () used in the PMCH.  Else the UE shall usefor the PMCH and Table 7.1.7.1-1 to determine the modulation order () and TBS index () used in the PMCH. 
The UE shall then follow the procedure in subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to. The UE shall set the redundancy version to 0 for the PMCH.

## 11.2	UE procedure for receiving MCCH change notification

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the M-RNTI, the UE shall decode the PDCCH according to the combination defined in Table 11.2-1.

Table 11.2-1: PDCCH configured by M-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 1C | Common |

The 8-bit information for MCCH change notification [11], as signalled on the PDCCH, shall be delivered to higher layers.

# 12	Assumptions independent of physical channel

A UE shall not assume that two antenna ports are quasi co-located unless specified otherwise.

A UE may assume the antenna ports 0 – 3 of a serving cell are quasi co-located (as defined in [3]) with respect to delay spread, Doppler spread, Doppler shift, average gain, and average delay.

For the purpose of discovery-signal-based measurements, a UE shall not assume any other signals or physical channels are present other than the discovery signal.

If a UE supports discoverySignalsInDeactSCell-r12, and if the UE is configured with discovery-signal-based RRM measurements on a carrier frequency applicable for a secondary cell on the same carrier frequency, and if the secondary cell is deactivated, and if the UE is not configured by higher layers to receive MBMS on the secondary cell, the UE shall, except for discovery-signal transmissions, assume that PSS, SSS, PBCH, CRS, PCFICH, PDSCH, PDCCH, EPDCCH, PHICH, DMRS and CSI-RS may be not transmitted by the secondary cell until the subframe where an activation command is received for the secondary cell.

# 13	Uplink/Downlink configuration determination procedure for Frame Structure Type 2

If the UE is configured with a SCG, the UE shall apply the procedures described in this clause for both MCG and SCG

-	When the procedures are applied for MCG, the terms ‘secondary cell’, ‘secondary cells’ , ‘serving cell’, ‘serving cells’ in this clause refer to secondary cell, secondary cells, serving cell, serving cells belonging to the MCG respectively.

-	When the procedures are applied for SCG, the terms ‘secondary cell’, ‘secondary cells’, ‘serving cell’, ‘serving cells’ in this clause refer to secondary cell, secondary cells (not including PSCell), serving cell, serving cells belonging to the SCG respectively. The term ‘primary cell’ in this clause refers to the PSCell of the SCG.

For each serving cell

If the UE is not configured with the higher layer parameter EIMTA-MainConfigServCell-r12,

-	the UE shall set the UL/DL configuration equal to the UL/DL configuration (i.e., the parameter subframeAssignment) indicated by higher layers.

If the UE is configured by higher layers with the parameter EIMTA-MainConfigServCell-r12, then for each radio frame,

-	the UE shall determine eIMTA-UL/DL-configuration as described in subclause 13.1.

-	the UE shall set the UL/DL configuration for each radio frame equal to the eIMTA-UL/DL-configuration of that radio frame.

## 13.1	UE procedure for determining eIMTA-uplink/downlink configuration

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the eIMTA-RNTI, the UE shall decode the PDCCH according to the combination defined in Table 13.1-1.

Table 13.1-1: PDCCH configured by eIMTA-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 1C | Common |

The subframes in which the UE monitors PDCCH with CRC scrambled by eIMTA-RNTI are configured by higher layers.

For each serving cell,

-	if T= 10,

-	if the UE detects PDCCH with CRC scrambled by eIMTA-RNTI in subframe 0 of a radio frame m or if the UE detects PDCCH with CRC scrambled by eIMTA-RNTI in a subframe other than subframe 0 of a radio frame m-1,

-	the eIMTA-UL/DL-configuration for radio frame m is given by the UL/DL configuration indication signalled on the PDCCH as described in [4],

-	the UE may assume that the same UL/DL configuration indication is indicated by PDCCH with CRC scrambled by eIMTA-RNTI in subframe 0 of radio frame m and in all the subframes other than subframe 0 of radio frame m-1 in which PDCCH with CRC scrambled by eIMTA-RNTI is monitored,

-	otherwise

-	the eIMTA-UL/DL-configuration for radio frame m is same as the UL/DL configuration (i.e., the parameter subframeAssignment) indicated by higher layers;

-	if T is a value other than 10,

-	if the UE detects PDCCH with CRC scrambled by eIMTA-RNTI in a subframe in radio frame mT/10,

-	the eIMTA-UL/DL-configuration for radio frames {mT/10+1 , mT/10+2,…. (m + 1)T/10} is given by the UL/DL configuration indication signalled on the PDCCH as described [4],

-	the UE may assume that the same UL/DL configuration indication is indicated by PDCCH with CRC scrambled by eIMTA-RNTI in all the subframes of radio frame mT/10 in which PDCCH with CRC scrambled by eIMTA-RNTI is monitored,

-	otherwise

-	the eIMTA-UL/DL-configuration for radio frames {mT/10+1 , mT/10+2,…. (m +1) T/10} is same as the UL/DL configuration (i.e., the parameter subframeAssignment) indicated by higher layers.

where T denotes the value of parameter eimta-CommandPeriodicity-r12.

For a serving cell , if subframe i is indicated as uplink subframe or a special subframe by higher layer parameter eimta-HARQ-ReferenceConfig-r12, the UE is not expected to receive a PDCCH with CRC scrambled by eIMTA-RNTI containing an UL/DL configuration for serving cell  that would indicate subframe i as a downlink subframe.

For a serving cell , if subframe i is indicated as downlink subframe or a special subframe by higher layer parameter subframeAssignment, the UE is not expected to receive a PDCCH with CRC scrambled by eIMTA-RNTI containing an UL/DL configuration for serving cell  that would indicate subframe i as an uplink subframe.

For a serving cell , a UE is not expected to be configured with parameter eimta-HARQ-ReferenceConfig-r12 if a subframe indicated as an uplink subframe by eimta-HARQ-ReferenceConfig-r12 is not indicated as an uplink subframe by the UL-reference UL/DL configuration.

If UE is not configured with the parameter EIMTA-MainConfigServCell-r12 for any activated serving cell, the UE is not expected to monitor PDCCH with CRC scrambled by eIMTA-RNTI.